GEL (version of 2/17/89)

February 1989  John R. Thompson

("README.DOC added 2/22/89 by Spencer Yeh)

INTRODUCTION

John R.  Thompson's GEL is an IBM-compatible program which aids in

calculating DNA fragment sizes.

PROGRAM GEL takes a set of standard DNA fragment sizes and mobilities and

predicts the sizes of unknown fragments.  GEL applies a least squares fit to

the reciprocal relationship between mobility and fragment length originally

detailed by Southern (Anal. Biochem 100,319-323[79]).  This program is a Turbo

Pascal(tm) implementation of the FORTRAN program described by Schaffer and

Sederoff (Anal. Biochem 115,113-122[81]).  Please refer to those references

for complete details of the calculations performed by this program.

This version allows the user to edit individual standards and

recalculate without the need to re-enter unchanged data.  Additionally,

standard fragment data may be stored in and read from a disk file.  This menu

driven looping structure makes it easy to exclude obviously aberrant data and

re-calculate.

Thompson's GEL program has a similar function to the IntelliGenetics

SIZER program which is available on BIONET.  (Note: John Thompson's GEL

program is not related to the IntelliGenetics GEL program which is also

available on BIONET.)

CONTACT ADDRESS

For questions about the program or suggestions for future improvements

please contact:

John R. Thompson

7105 Hana Rd.

Edison, NJ 08817

e-mail:  woolford.thompson@bionet-20.bio.net

SYSTEMS SUPPORTED

An IBM-compatible computer running MS-DOS is needed.  If hardcopy is

desired, an IBM-compatible printer is required.

AVAILABILITY

GEL is available for downloading from BIONET in the directory

<PC-SOFTWARE.GEL> or by postal mail from the BIONET Lending Library.  If

you would like to receive GEL by mail, please send a stamped,

self-addressed return envelope along with a formatted diskette (specify

capacity) and your request to:

BIONET Administrator

BIONET/IntelliGenetics

700 East El Camino Real, Suite 300

Mountain View, CA 94040

tel.: (415) 962-7337

SOURCE CODE

Source code written in Turbo Pascal is available in compressed form in

the file GELSRC.ARC.  Use an "ARC"-compatible dearchiving program to

restore the GEL.PAS source code file.

PROGRAM FILES before de-ARCing  (BIONET diskette version)

README.DOC      This documentation file.

GEL.EXE		Self-extracting archive file for the executable and

documentation.  (46 Kb).

GELSRC.ARC	Archived Turbo Pascal source code.  (21 Kb).

PROGRAM FILES before de-ARCing  (BIONET downloadable version)

README.DOC      This documentation file.

GEL.UUE		Archived and uuencoded file for the executable and

documentation.  (46 Kb).

GELSRC.UUE	Archived and uuencoded Turbo Pascal source code.  (30 Kb).

PROGRAM FILES after de-ARCing (Approx. 42 Kb total)

GEL.DOC		Documentation file

GEL.EXE		Executable file

DOCUMENTATION

The program is documented in the file GEL.DOC in addition to containing

internal help messages.

STARTING THE PROGRAM

On the BIONET diskette version, the GEL archive file GEL.EXE is a

self-extracting archive, whereas the ".uue" files in the downloadable

version require both the UUDECODE program and an "ARC"-compatible

dearchiving program such as PKUNPAK to restore the executable file.

Self-dearchiving files:

De-archive the program by "running" the archive file and specifying the

drive and directory path where you want the program installed.  E.g., to

install GEL in the \GEL directory of the c: drive, you should type:

>GEL c:\gel

".UUE" files:

First decode the ".uue" file:

>UUDECODE gel.uue

Then dearchive the resulting ".arc" file to an appropriate directory by

using PKUNPAK (or compatible program):

>PKUNPAK gel.arc c:\gel

Once installed, CD to the appropriate directory and then start the

program by typing its name at the MS-DOS prompt:

>GEL

MAIN MENU FOR PROGRAM:

GEL : MAIN MENU

Select Function By First Letter

Documentation Screen

Input standard fragments and mobilities

Get standards from a disk file

Edit standard curve data

Save standard curve data to disk

Calculate standard curve

Unknown fragment size calculation

Output Device Selection [Currently = SCREEN]

Quit the Program and Exit to DOS

?

=========================================================================

SAMPLE OUTPUT

STANDARD CURVE  : Program GEL  :  Version 4/14/87 JRT

Std size  : Mobility : Pred. Size : Deviation :  %DEV  :  C(i)

5241.000       3.03     5245.058      -4.058    -0.08     26082.97

5070.000       3.11     5046.383      23.617     0.47     26173.47

4257.000       3.46     4280.093     -23.093    -0.54     26012.10

3510.000       3.89     3519.436      -9.436    -0.27     26057.64

1995.000       5.17     1979.983      15.017     0.75     26176.19

1880.000       5.30     1863.921      16.079     0.86     26183.95

1584.000       5.63     1592.661      -8.661    -0.55     26045.72

1350.000       5.95     1357.550      -7.550    -0.56     26049.75

941.000       6.60      948.387      -7.387    -0.79     26045.94

843.000       6.80      837.840       5.160     0.61     26131.94

M0 = -0.17186        L0 = -2905.20        C(Avg) = 26095.97

SD(Dev) = 16.448                SD(C) =  64.39

===============================================================

UNKNOWNS SIZE PREDICTION

Frag1: Size = 5322.14 For a Dist. of   3.00

Frag2: Size = 2140.56 For a Dist. of   5.00

=========================================================================