ZAP may be used to permanently change the default  vec-

tors and CSR's for the stand alone versions of BAD, DSC, and

PRESRV.  TKB should first be run to generate a load  map  of

the  utility from the .STB file provided on the distribution

kit.

For example, to change the MT vector in DSCS8 from  320

to the standard 224:

RUN ZAP

DSCS8.SYS/AB

3:224/

22434 (must equal $MMINT from map)

25610 (equals $MTINT from map)

X

CSR addresses can be changed by  modifying  the  proper

SCB entry e.g.  $MT0+12 for MT: