431^H01      301

Article 4301 (18 more) in rec.woodworking:

From: roche@cs.rochester.edu (James Roche)

Subject: Frequently Asked rec.woodworking Questions (last changed 8/8/91)

Message-ID: <1991Sep3.133324.8400@cs.rochester.edu>

Date: 3 Sep 91 13:33:24 GMT

Reply-To: roche@cs.rochester.edu

Organization: University of Rochester Computer Science Department

Lines: 257

I have been reading and archiving rec.woodworking since it's inception

as net.rec.wood back in 1984. Below are some of the topics that seem to

come up frequently.

If you have any constructive comments please let me know. If there is

something you would like to see added please pass it on to me.

TABLE OF CONTENTS:

1). Should I buy a table saw or a radial arm saw?

2). Which type of dado blade should I buy, the dial (wobble type) or

the stacking (chipper type)?

3). How do I cut the perfect dado if both types of dado blades have

shortcomings?

4). Should I buy a Sears blurfl?

--MORE--(9%)        5). Should I buy a Taiwanese clone blurfl?

6). How do I remove paint?

7). Should I use a hot melt glue gun for my next project?

8). Where can I get plans for the New Yankee Workshop projects?

9). What is the best woodworking magazine?

10). What is a board foot?

11). What is the correct way to handle the glue squeeze out problem?

12). What books should I purchase to learn about various aspects of

woodworking?

13). How do I finish toys so that they are non-toxic?

14). What size drill bit do I use for a wood screw?

15). How do I finish the edge of plywood?

1). Should I buy a table saw or a radial arm saw?

Table saws work best for ripping. Radial arm saws work best for

crosscutting. However, both saws will perform both tasks.

Ripping on the radial arm saw is difficult because it is necessary

to push the lumber under the motor housing.  This problem can be

minimized by use of pushsticks.

--MORE--(17%)        Crosscutting on the table saw is difficult because it is tough to

keep a board much longer than 4 ft square to the blade.  This

problem can be minimized by building a good sliding panel cutter.

Some people say that the radial arm saw is more dangerous than

the table saw because the blade is exposed above the work surface.

The radial arm saw tends to lift the work off the work surface

when the saw is used for ripping, and can pull itself into the work

(resulting in binding of the saw or serious injury to a careless

operator) in the crosscut position.  The table saw tends to push

the work down onto the work surface. Both saws have the capability

to kick back the work.

Both machines are very dangerous and should be treated with

much respect.

2). Which type of dado blade should I buy, the dial (wobble type) or

the stacking (chipper type)?

The wobble type is very simple to use with infinite settings

between approximately 1/4 inch and 13/16 inch. It does not produce

true right angle cuts because of the design.

--MORE--(27%)        The chipper type doesn't have the same infinite setting for width.

You can achieve nearly infinite settings by adding shims with

thicknesses of 1/32, 1/64, 1/128, etc.  It will cut a square bottom

on the dado, but it will also leave 2 grooves on the edges of the

dado. The reason for this is that the blades are slightly larger

than the chippers.  The larger blades are to reduce the splintering.

The blades can be reground to be equal to the size of the chippers

at the possible cost of increased splintering. Some say the grooves

are a benefit because they provide relief for gluing joints.

Many people claim that the wobble type is easier to set up.

The June 1991 Fine Woodworking further studies the features of

the various types of dado blades.

3). How do I cut the perfect dado if both types of dado blades have

shortcomings?

Your best bet would be to cut it with a router.

4). Should I buy a Sears blurfl?

Most people agree that the Sears stationary power tools sold today

--MORE--(36%)        aren't the same quality as the Sears tools sold 20 years ago.

It can be argued that if you can't afford to buy a Delta, then

you should be looking at one of the Taiwanese clones rather than

looking at Sears.

5). Should I buy a Taiwanese clone blurfl?

It depends on how much money you have. If you can afford the

Delta blurfl you should probably get it. Buyers should be

wary that not all Taiwanese clones are quality machines. The

general consensus is that Grizzly has good quality control.

It should be noted that some of Delta tools are now being made

in Taiwan. One would hope that Delta quality control is better than

some of the cheaper imports.

6). How do I remove paint?

There are many ways to strip paint from wood.

Paint can be removed by scraping and/or sanding.

Paint can be removed by using chemical paint removers.

--MORE--(44%)        Paint can be removed by using heat.

Paint can be removed by sandblasting.

Paint can be removed by a new product known as Peel-Away.

If you know of another way to remove paint please feel free to pass

the information along.

7). Should I use a hot melt glue gun for my next project?

The general consensus is that hot melt glue is not adequate

for woodworking projects. However, hot melt glue guns can

have a place in the shop. Many people like to use them for

tacking items together such as when building forms or jigs.

8) Where can I get plans for the New Yankee Workshop projects?

Plans for any of Norm's projects can be ordered for $7.50 from

(project name)

New Yankee Workshop

P.O. Box 645

Bedford, MA  01730

--MORE--(51%)        Videos, which include a copy of the plans, cost $24.95 plus shipping

and handling, from 800-272-0280.  Both the address and phone number

are given at the end of each show.

Most of the projects from the first two seasons are in the two

New Yankee Workshop books.  For the workbench, one important

dimension is *not* given in the book, though it can, I think,

be calculated.

9) What is the best woodworking magazine?

There are many good woodworking magazines. Two that are frequently

recommended in this group are Fine Woodworking and Woodsmith.

Fine Woodworking is a bit on the artsy side and more for the

experienced woodworker. It does not get into the small details

of a project.

Woodsmith provides much more details for projects. Woodsmith is

well suited for both the amateur and experienced woodworker.

10) What is a board foot?

--MORE--(59%)        A board foot is a common unit used in the measurement of wood.

It is equal to 1 foot length x 1 foot width x 1 inch thick.

It should be noted that the thickness is nominal thickness.

After drying and surfacing the usual thickness of a 1 inch

board is 13/16.

A board 10 feet long x 1 foot wide x 2 inches thick would be

equal to 20 board feet.

11) What is the correct way to handle the glue squeeze out problem?

Use the right amount of glue.  The (obvious) danger is a glue

starved joint.

Wipe off the excess glue immediately with a damp sponge or paper

towel.  This method gets mixed reviews.  Some claim the water-glue

mixture will soak into the wood and show up when the piece is finished.

Others say that this is not a problem.  The effectiveness of this

method probably depends on the type of wood and finish that are used.

Allow the glue to harden somewhat (1/2 - 2 hrs) and then

chisel/scrape it off.  Some recommend removing the glue after it

begins to film over.

--MORE--(68%)

Either finish the pieces ahead of time or apply paste wax.  This

should prevent the glue from sticking.  The problem with this is

removing the paste wax prior to finishing.

Use a plastic drinking straw cut at 45 degrees to scoop the glue

out of the inside corner.  As the straw fills up, it can be trimmed to

provide a fresh surface.

12) What books should I purchase to learn about various aspects of woodworking?

GENERAL WOODWORKING

Cabinetmaking and Millwork - John L. Feirer

New Yankee Workshop - Norm Abrams

INTRODUCTORY WOODWORKING

Basic Woodworking - Sunset Books

JOINERY

Tage Frid Teaches Woodworking: Joinery Tools and Techniques

- Tage Frid

Tage Frid Teaches Woodworking: Shaping, Veneering, Finishing

- Tage Frid

--MORE--(75%)

TOOLS

Router Jigs and Techniques - Patrick Spielman

200 Original Shop Aids & Jigs for Woodworkers -Rosario Capotosto

WOOD

Understanding Wood - R. Bruce Hoadley

13) How do I finish toys so that they are non-toxic?

a.  Behlens Salad Bowl Finish.  This product has been approved by

the FDA for use on objects that will come in contact with

food.  It produces a nice, semi-gloss finish.

b.  Any vegetable oil. Could become rancid after a period of time.

c.  Walnut oil.  It reacts with the air and hardens into a true

finish.  It works particularly well when the oil is heated

and the item is dipped into the warm oil. Available at health

food or large grocery stores.  Don't buy the gourmet stuff!

d.  Mineral oil or vaseline.

e.  Water-based polyurethanes.  These are new products which are

very different from the more familiar oil-based

polyurethanes.  They are totally non-toxic, dry quickly, and

--MORE--(84%)            have no strong odors when applying.

f.  Pure Tung Oil. It has no driers or solvents. It is essentially

just a vegetable oil but produces a nice finish that won't go

rancid. Use only Tung Oil that is "pure".

f.  Leave items unfinished.

14) What size drill bit do I use for a wood screw?

Screw Gage    Shank    Shank     Pilot     Pilot

Number        Hole    Size      Soft Wd   Hard Wd

0         1/16    .060      1/64      1/32

1         5/64    .073      1/32      1/32

2         3/32    .086      1/32      3/64

3         7/64    .099      3/64      1/16

4         7/64    .112      3/64      1/16

5         1/8     .125      1/16      5/64

6         9/64    .138      1/16      5/64

7         5/32    .151      1/16      3/32

8         11/64   .164      5/64      3/32

9         3/16    .177      5/64      7/64

10         3/16    .190      3/32      7/64

11         13/64             3/32      1/8

12         7/32    .216      7/64      1/8

--MORE--(94%)      14         1/4     .242      7/64      9/64

16         17/64   .268      9/64      5/32

18         19/64   .294      9/64      3/16

20         21/64   .320      11/64     13/64

15) How do I finish the edge of plywood?

a) Wood tape. It comes in 2 forms, one that can be ironed on, and

one that can be contact cemented on. The tape is approximately

1 inch wide, and can be trimmed with a plane, router, or knife.

b) Glue strips of wood, either purchased or cut from lumber.

--

Jim Roche

roche@cs.rochester.edu

University of Rochester Computer Science Department Rochester, NY 14627

End of article 4301 (of 4320)--what next? [npq] 4302

Article 4302 (17 more) in rec.woodworking:

From: roche@cs.rochester.edu (James Roche)

Subject: Frequently Requested rec.woodworking Addresses (last changed 8/1/91)

Message-ID: <1991Sep3.133454.8500@cs.rochester.edu>

Date: 3 Sep 91 13:34:54 GMT

Reply-To: roche@cs.rochester.edu

Organization: University of Rochester Computer Science Department

Lines: 434

Some of the more popular addresses/phone numbers used by woodworkers are

included below. If you want to see some addresses added to this list please

let me know.

This is compiled from messages that have been posted to the group over the

past few years, and from vendors that I have used. This is not an endorsement

of any of the vendors listed.

Much of the text of this message came from a similar posting a while back

from nivek@rover.ri.cmu.edu (Kevin Dowling). My intent is to keep it

as up to date as possible and make it a regular monthly posting.

Contributors included:

mauney@cscadm.ncsu.edu (Jon Mauney)

--MORE--(6%)        Robert Rifkin <rr%linus@mitre-bedford.ARPA>

bill daniels {hplabs!csun,gatech!emard}!fedeva!wrd3156

brackett@dvinci.DEC  [from a post a few years ago]

*******************************************************************

U.S General                             Ordering: 1-800-645-7077

100 Commerical Street                   Customer service: (516) 349-7275

Plainview, NY 11803

**** APPARENTLY NO LONGER IN THE MAIL ORDER BUSINESS. ****

Typical mail-order discount tool catalog. Name brand tools.

Big catalog. Seem to be resonable prices. 24 hour phone.

Seven Corners Ace Hardware              Ordering: 800-328-0457

(Tools on Sale)

216 West 7th St.                        In MN: (612) 224-4859

St Paul, MN 55102                       Hours: M-F 8-5:30 CST

Big catalog. Emphasis on portable power tools but they carry a wide

range of tools and accessories.  Prepaid freight. "..good experiences.

Foley-Belsaw                            Orders: 800 468-4449

--MORE--(12%)6301 Equitable Road                     In MO 800 892-8789

Box 419593

Kansas City, MO 64141

Large shop equipment: Planers, table saws, combination machines,

even a sawmill! They send out several free catalogs a year and have

a number of special offers throughout the year.

Grizzly Imports

2406 Reach Road                 POB 2069

Williamsport, PA 17701          Bellingham, WA 98227

(717) 326-3806                  (206) 647-0801

M-F 8:30-5:30                   M-F 8:30-5:30

Sat 8:30-1

Shop Machinery - Imports.

Hida Tool & Hardware Company                    (415) 524-3700

1333 San Pablo Avenue           Store hours: T-S 9-5, Sun 11-4 Closed Mon

Berkeley, CA 94702

Catalog $3

Japanese Hand Tools. Excellent quality saws, planes, chisels, knives

etc. Prices are commensurate with the quality but reasonable. Hida is

across from REI in Berkeley.

--MORE--(17%)Woodcraft                               (800) 225-1153

41 Atlantic Avenue                      From MA, HA, and AL call (617) 935-5860

PO Box 4000                     M-F 8:30-6 EST Answering machine at other times

Woburn, MA 01888

Store at 313 Montvale Ave Woburn MA

Wide range of tools for craftsman. Nice catalog, good prices, excellent

delivery.

Woodworkers Supply of New Mexico        (800) 645-9292 8:30-5 MT M-F

5604 Alameda Place NE                   (800) 321-9841

Albuquerque, NM 87113                   (505) 821-0500

Wide range of tools. Delta etc. Power tool emphasis.

"...fast, reliable service"

"Small but well rounded selection of hand tools, books etc."

Stone Mountain Power Tool Corporation   (800) 262-9599

POB 2825                                In GA (404) 446-8390

Norcross, GA

Woodworking and Construction Tools. Name brands. Reasonable prices.

Brookstone                              Orders: (603) 924-9541 (24 hours)

127 Vose Farm Road                      Customer service: (603) 924-9511 9-5 EST

--MORE--(23%)Peterborough, NH 03458

Unusual tools with a number of gadgets. Nice catalog. Brookstone

has a numnber of stores around the country. You can sometimes find

specific items for less elsewhere.

Hiller Hardware                         (803) 779-3131 M-F 9-5, Sat 9-1

1411 Assembly Street

POB 1762

Columbia, SC 29202

Power Tools mostly. Delta, Porter-Cable, Ryobi, Bosch, Inca, Skil,

Makita, Milwaukee, Freud, and others.

Garrett Wade                            Orders: (800) 221-2942

161 Avenue of the Americas              Customer service: (212)807-1155 M-F 9-5

New York, NY 10013

Catalog $4

One of the finest catalogs. A woodworker's bible of tools. GW also

has a separate hardware catalog.

"The prices are good considering the lavish catalog."

Bob Morgan's Woodworking Supplies       (502) 456-2545

1123 Bardstown Road

--MORE--(28%)Louisvill, KY 40204

Hardwoods, veneers, veneering tools and hand tools.

Leichtung Workshops                     (800) 321-6840 (24 hours)

4944 Commerce Parkway                   In OH (216) 831-2555

Cleveland, OH 44128

Kind of like Brookstone but gadget/tool ratio is much higher.

The Wood Store                          (717) 297-4188

POB 125

Troy, PA 16947

(717) 297-4188

Hardwoods, carving blocks, basket strips, dollhouse lumber.

Cryder Creek Wood Shoppe                (800) 642-9663

Box 19                                  In NY (607) 356-3303 9-4 EST

Whitesville, NY 14897

Woodturner's supplies

Frog Tool Co                            (312) 648-1270

700 W. Jackson Blvd.

Chicago, Il 60661

Hundreds of woodworking books. All kinds, all woodworking subjects.

--MORE--(32%)

R. Sorsky                               (800) 345-4447

3845 N. Blackstone,                     In CA (209) 227-2901

Fresno, CA 93726

Woodworking books. Lots.

The Woodworker's Store                  Orders: (612) 428-2199 M-F 8-8 CST

21801 Industrial Blvd.                  Customer service: (612) 428-2899

Rogers, MN 55374

WWS has 8 stores around the country also. Hardwood, veneers.

Very good selection of hardware. Hinges, slides, locks, catches,

pulls and other accessories.

AMT (American Machine and Tool Company)         (215) 948-0400

Fourth Avenue and Spring Street

Royersford, PA 19468

Name is somewht of a misnomer since much if not all the tools

are imports. Similar to Grizzly catalog with more hand tools.

Cascade Tool Co.                        (800) 235-0272

Box 848                                 In WA 236-027M-F 7-7 PT

Mercer Island, WA

Router and shaper bits. All kinds.

--MORE--(38%)

Craftsman Wood Service Company          (708) 629-3100

1735 W. Cortland CT

Addison, IL 60101

(312) 629-3100

Small format hand tools and related supplies. Veneer,hard wood,turning

blocks,excellent selection of hardware, books  and plans and toys.

Mason & Sullivan                        (508) 778-0475 M-F 8-5

586 Higgins Crowell Road                (508) 778-0477 (after hours)

West Yarmouth,                          Customer service:(508) 778-1056

Cape Cod, MA 02673

Clockbuilding related items. Kits, tools, the works.

Klockit                                 (800) KLOCKIT 556-2548 M-F 8-8 Sat 8-4

POB 629

Lake Geneva, WI 53147

Clock Kits. Mason & Sullivan catalog looked better.

The Fine Tool Shops                     (800) 533-5305

170 West Road                           In NH 443-0409

POB 7091

--MORE--(42%)Portsmouth, NH 03801

Good catalog, selection and prices. "...fast, reliable service"

Sears

Sears Tower

Chicago. IL 60606

...and just about every other city in the country.

Power and Hand Tools Specialog.

Constantines                            (800) 223-8087

2050 Eastchester Road                   In NY (800) 822-1202

Bronx, NY 10461

Probably the oldest woodworking company in the country (176 years)

Wide selection of tools, woods, veneers, and hardware.

John Stortz and Son                     (215) 627-3855

210 Vine Street

Philadelphia, PA 19106

This catalog may not quite fit into this catagory but Storz is

one of the only manufacturers of these kinds of tools. They specialize

in the production of tools for masonry, roofing, shipbuilding, and

cooperage. They normally sell to the trades and don't take chargecards.

--MORE--(47%)JET Woodworking Machinery and Supplies          (800) 243-8538

POB 1477                                        (206) 572-5000

Tacoma, WA 98401

Similar to Grizzly. Warehouses throughout the country.

Advanced Machinery Sales (AMS)          (302) 322-2226

POB 312

New Castle, DE 19720

Selling Hegner, Felder, AMI

J. Philip Humfrey                       (800) 387-9789

3241 Kennedy Road, Unit 7,              In Canada (416)293-8624

Scarborough, Ontario M1V 2J9

(800) 387-9789 In Canada (416)293-8624

Woodworking Machinery - Shop equipment, General, Excalibur,

Concord, Kraemer.

Trendlines                              Orders: (800) 767-9999

375 Beacham Street                            (800) 343-3248  Answering machine

Chelsea, MA 02150                       Customer service: (617) 884-8882

Woodworking tools and supplies. Good prices.

--MORE--(52%)Hartville Hardware                      (800) 345-2396 M-S 8:30-5

940 W. Maple Street

Hartville, OH 44632

Brand name tools. Big discount place. No catalog available.

Cherry Tree Toys                        (614) 484-4363 (24 hours)

POB 369,

Belmont, OH 43718

Wood toys. Kits, parts, plans, books.

Pro-Shop                                (312) 832-3803

Box 721

Elmhurst, IL 60126

Woodworking machinery. Imports.

Wilke Machinery Company                 (717) 846-2800

120 Derry Ct.

York, PA 17402

Woodworking machinery. Imports. Large selection.

Sporty's Tool Shop              Orders: (800)543-8633 M-F 9:30-8, Sat 11-3

Batavoia, OH 45103              Customer service: (513) 732-2411

Some tools. Mostly random stuff for house and yard.

--MORE--(56%)

Lee Valley Tools Ltd.

P.O. Box 6295, Station J

Ottawa, Ont.

K2A 1T4

"Fine woodworking tools. Several stores.  They have a main catalogue, a

couple of supplements a year, and a hardware catalogue,  Even if you

don't understand tools at all, the main catalogue is worthwhile

getting as a coffee table book: glossy paper and beautiful presentation.

It makes a wonderful read. They have everything from instrument-maker's

to log builder's hand tools, Their bottom-of-the-line quality is quite

servicable, and their top-of-the-line tools are Art, with accompanying

prices.  The bottom line is: good selection, high quality, reasonable

prices, *excellent* service. They design and manufacture many quality

tools under the VERITAS label.

Factory Lumber Outlet

200 Shrewsbury St

Boylston

"Large number of hand tools, books and plans.  Separate wood catalog.

--MORE--(62%)Xylophile's Co                          800-354-9083

138 E Louden Ave

Lexington KY 40505

"fast reliable service" [no other info]

Highland Hardware                       (800) 241-6748

1045 N Highland Ave. NE                 (404) 872-4466

Dept F  Atlanta GA 30306

Good source for glues, finishes, etc.  Has Delta tools, good router

and bisquit joiner selections.  Carries MiniMax and Robland

stationary tools also.

Tools Etc.

Dept 458411 510 East Main St

Louisville KY 40202

[No other information]

Bratton Machinery & Supply Inc.

1015 Commercial ST.

PO Box 20408

Tallahassee FL 32316

[no other information]

--MORE--(66%)Berea Hardwoods                         (216) 243-4452

125 Jacqueline Drive,

Berea OH 44017

"Specializing in unusual woods, but also a good source for

more mundane species.  The proprietor is fun to talk to."

Craftman's Corner, Inc.                  515-265-3239

4012 N.E. 14th Street

P.O. Box AP

Des Moines, Iowa  50302

[no other information]

The Taunton Press                       (800) 888-8286

63 South Main St.

P.O. Box 355

Newtown, Conn.  06470-9971

Publisher of Fine Woodworking, Fine Homebuilding and numerous

quality books on woodworking and homebuilding

Woodsmith                               (800) 444-7002

P.O. Box 10740

Des Moines, Iowa  50347-0740

Publisher of Woodsmith magazine

--MORE--(70%)

Industrial Abrasives                    (800) 428-2222

642 N 8th Street                        In PA (800) 222-2292

Box 14955

Reading PA 19612

Sandpaper, sanding drums, assorted abrasives.

The Sanding Catalog                     (800) 228-0000

P.O. Box 5069

Hickory, NC 28603-5069

Sandpaper and assorted abrasives.

Econ-Abrasives                          (800) 367-4101

PO Box 865021                           In TX (214) 377-9779

Plano, TX, 75086-5021                   M-F 7:30-5:00 CST

They carry a wide range of sandpaper (belts, sheet, discs,

rolls, drums).  Good prices, eg: 3x21 belts @ .77,

4x24 belts @ .94

Minimum order around $20, $4.25 shipping, take MC/VISA.

The people on the phone KNOW sandpaper - they can help with

most any abrasives question.

--MORE--(75%)Wood Finishing Supply Co. Inc           315-986-4517

1267 Mary Drive

Macedon, NY 14502

All types of finishing supplies. The catalog also provides some

good informationon how to use the finishes.

WoodFinishing Enterprises               414-871-0440

2438 North 48th Street

Milwaukee, Wis. 53210

All types of finishing supplies.

Whole Earth Access                      (800)829-6300

822 Anthony Street                      9AM to 6PM PST, 7 days a week

Berkeley, CA  94710                     FAX (415)845-8846

Hand Tools and Power Tools.  Sorted by type of equipment - not brand.

Bosch, Porter Cable, Makita, Elu, AEG, Ryobi, Milwaukee, Hitachi,

Skil, Panasonic, B&D, DeWalt and Delta.  Low to average prices.

48 pages

Northern Hydraulics                     (800)553-5545  24 hrs, 7 days

P.O. Box 1499                           Minn: (612)894-8310

Burnsville, MN  55337-0499              FAX (612)894-8310

All kinds of stuff - some woodworking.  Engines, compressors,

--MORE--(80%)        lawn equipment, chain saws, wood spliters, hydraulic fittings,

air tools, mechanics tools, tool boxes, work benches, playground

sets, sun glasses, leather jackets, tool kits, winches, trailers,

jacks, wheels, tires, tractors and even watches.

Woodworking power tools include Milwaukee, Makita, Bosch, Ryobi,

Porter Cable, Jepson, B&D, Skil, Wen and Delta.  Mostly portable.

Sorted by tool type.  Average prices.  136 pages.

Schlosser Tool & Machinery              (800)627-7297

301 Bryant                              CO (303)636-1311

Denver, CO  80219                       FAX (303)937-9267

Mostly power tools - industrial, stationary and portable.  Includes

Skil, Porter Cable, Delta, Makita, Hitachi, Blum, Bosch, Ryobi,

DeWalt, Beisemeyer, Powermatic and others.  Some hand tools like

Sorby.  Sorted by tool type.  Average to high prices.  75 pages.

Williams Tool & Hardware Supply         (800)338-6668       8AM-6PM CST M-F

2017 White Settlement Road              TX (817)335-4749    9AM-5PM CST Sa

Fort Worth, TX  76107                   FAX (817)877-3150

Stationary and portable power tools, hand tools and supplies.

Includes Delta, Ryobi, Porter Cable, Hitachi, Bosch and Elu

power tools.  Sorted by manufacturer.  Low prices.  32 pages.

--MORE--(88%)

Chown Hardware & Machinery              (800) 547-1930

333 N.W. 16th Ave                       Oregon (800) 452-7634

Portland, OR 97209                      Local (503) 243-6500

This company has been around a long time, since 1879.

Nice catalog, fair prices, and a flyer that they send out

every month with the current specials.

Tarheel Filing Co                       (800) 322-6641

3400 Lake Woodard Drive

Raleigh, NC 27604

Stationary power tools.

Philipps Brothers Supply                (800) 238-4466 (NY)

2525 Kensington Ave.                    (800) 544-4466 (USA)

Amherst, NY  14226-4999

A large list of power tools for woodworkers and construction

professionals.  Prices seem pretty good, shipping is usually included.

Tool Crib of the North                  (800) 358-3096

Formerly Acme Electric. Good power too selection. All magor brands

including Delta, Elu, B&D, Skil, Makita, Milwaukee, Powermatic,

Bosch, Freud, General, Hitachi. Free freight.

--MORE--(94%)

Woodhaven                               (800) 344-6657

They make router accessories (tables, jigs, fences, adjustment

handles, inlay bushings, etc).  Prices ok (not great), quality is

high (table inserts are of 3/8" Lexan, for instance).

Bridge City Tool Works

Squares and straightedges.  Very high prices and quality.

Shopsmith                               (800) 543-7586

Stores all over the US.

Eagle America                           (216) 286-9334

PO Box 1099                             (216) 286-7643 (FAX)

Chardon, Ohio 44024                     (800) 872-2511 (orders)

Router Bits

MLCS Ltd.                               (800) 533-9298

P.O. Box 4053

Rydal, PA  19046

Router Bits

--MORE--(98%)Warren Tool Co.                         (914) 876-7817

Rt. 1 Box 14-A

Rhinbeck, NY 12572

Carving tools (gouges, parting tools, tec) with interchangable

bits/handles.

--

Jim Roche

roche@cs.rochester.edu

University of Rochester Computer Science Department Rochester, NY 14627

End of article 4302 (of 4320)--what next? [npq] 4303

Article 4303 (16 more) in rec.woodworking:

From: roche@cs.rochester.edu (James Roche)

Subject: Frequently Requested Sources for Tool Reviews (last changed 9/3/91)

Message-ID: <1991Sep3.133616.8588@cs.rochester.edu>

Date: 3 Sep 91 13:36:16 GMT

Reply-To: roche@cs.rochester.edu

Organization: University of Rochester Computer Science Department

Lines: 147

Readers of this group often request reviews of particular tools. This article

is meant to be a pointer to tool reviews/comparisons. It is not a complete

list of sources for tool reviews. If anyone has any additions please send

me a note and I will try to add it in.

Air Compressors

Portable Air Compressors - Fine Homebuilding October 1988

Compressed Air Systems - Fine Woodworking June 1990

Air Tools

The WORKBENCH Guide to Air Tools - Workbench October 1988

Selecting a Spray Gun - Fine Woodworking June 1990

--MORE--(21%)Bandsaws

Shopping for Bandsaw Blades - Wood December 1989

Bandsaws - Wood October 1990

Belt Sanders

Portable Belt Sanders - Wood October 1990

Biscuit Joiners

The WORKBENCH Guide to Plate Joiners - Workbench April 1988

Plate Joiners - Fine Woodworking June 1989

Carvers

Power Carvers - Fine Woodworking April 1991

Circular Saws

Portable Circular Saws - Wood October 1990

Portable Circular Saws (AKA sidewinders) - Fine Homebuilding November 19 90

Clamps

The Smart Shoppers Guide to Clamps - Wood February 1991

Cordless Screwdrivers

--MORE--(34%)        The WORKBENCH Guide to Cordless Screwdrivers - Workbench April 1990

Cordless drill/drivers - Wood October 1990

Cordless Drills

Cordless drill/drivers - Wood October 1990

Cordless Wonders - Wood April 1991

Cut-Off Saws

Portable Electric Cut-Off Saws - Fine Homebuilding September 1990

Drill Bits

Buying Forstner Bits - Wood February 1991

Drills

The Great Electric Drill Shootout - Wood August 1990

Drill Presses

Drill Presses - Wood October 1990

Dust Collectors

The WORKBENCH Guide to Dust Collection Systems - Workbench February 1989

--MORE--(47%)Heat Guns

The WORKBENCH Guide to Heat Guns - Workbench February 1991

Jointers

Machines the Give You the Edge Quickly and Accurately - Wood October 198 9

Two New Jointers - Fine Woodworking February 1990

Jointers - Wood October 1990

Lathes

Lathes - Wood October 1990

The WORKBENCH Guide to Lathes - Workbench May 1991

Miter Saws

The WORKBENCH Guide to Power Miter Saws - Workbench June 1988

Getting an Angle on Compound Miters - Fine Homebuilding January 1990

Radial Arm Saws

Radial-arm Saws - Wood October 1990

Random Orbital Sanders

Tool Talk: Random Orbital Sanders - Popular Woodworking September 1991

--MORE--(62%)Routers

The WORKBENCH Guide to Routers - Workbench April 1989

Routers, Routers Everywhere - Wood February 1990

Plunge Routers - Wood October 1990

Sabersaws

What's New with Sabersaws - Fine Woodworking December 1990

Sawmills

The WORKBENCH Guide to Portable Sawmills - Workbench August 1989

Scrollsaws

Scrollsaw Blades - Wood April 1990

Scrollsaws - Wood October 1990

Scrollsaws over $500 - Wood September 1991

Shapers

Shapers - Wood December 1990

Shopsmith

Shopsmith vs. Total Shop - Wood June 1990

Tablesaws

--MORE--(75%)        How to Buy the Most Important Woodworking Machine in your Shop - Wood De cember 1989

Tablesaw Safety Devices - Fine Woodworking April 1990

Tablesaws - Wood October 1990

Thickness Planers

Portable Power Planes - Fine Homebuilding June 1989

The WORKBENCH Guide to Planers - Workbench February 1990

Thickness Planers under $500 - Wood April 1990

Portable Planer Survey - Fine Woodworking October 1990

Thickness Planers - Wood October 1990

Vises

Woodworking Vises - Wood September 1990

Mail Order

Mail Order Tools - Fine Homebuilding September 1990

Where to get back issues of magazines mentioned:

Fine Woodworking - Fine Homebuilding

The Taunton Press

--MORE--(90%)63 South Main Street

PO Box 5506

Newtown CT 06470-5506

Wood - Better Homes & Gardens

Meredith Corporation

1716 Locust St.

DesMoines IA 50336

Workbench

KC Publishing Inc

4251 Pennsylvania Ave

Kansas City, Missouri 64111

Popular Woodworking

Box 58279

Boulder, CO 80322

--

Jim Roche

roche@cs.rochester.edu

University of Rochester Computer Science Department Rochester, NY 14627

End of article 4303 (of 4320)--what next? [npq] 4307

Article 4307 (12 more) in rec.woodworking:

From: dsr@mitre.org (Douglas S. Rand)

Subject: Re: Thickness Planer Summary

Message-ID: <DSR.91Aug30103337@chickpea.mitre.org>

Date: 30 Aug 91 17:33:37 GMT

References: <1991Aug28.125132.191521@cs.cmu.edu>

<DSR.91Aug28134341@chickpea.mitre.org> <1991Aug29.162855.3782@amd.com>

Sender: news@linus.mitre.org (News Service)

Organization: The MITRE Corporation, Bedford, MA

Lines: 45

In-Reply-To: phil@brahms.amd.com's message of 29 Aug 91 16:28:55 GMT

Nntp-Posting-Host: chickpea.mitre.org

In article <1991Aug29.162855.3782@amd.com> phil@brahms.amd.com (Phil Ngai) write s:

dsr@mitre.org (Douglas S. Rand) writes:

>A better option for planing thin wood, and one I used when doing

>guitars, is to use a safety planer.  A safety planer will let you plane

>down to veneer thickness.

What's a safety planer?

--MORE--(43%)   I've seen a drum sander built for the purpose of producing thin wood.

Last weekend I needed some for splines and I "resawed" it on my table saw.

It wasn't great, but it was good enough.

Sorry,  guess I forgot that I might be talking nonesense for someone.  A

safety planer is a flat disk you chuck in a drill press with three cutters

on the edge.  It is difficult to come in contact with the cutters

accidentally as a lip protects them:

_________________________________

\

__|

|

-|

_____________________________   | < --- cutter

\__|

You can use them for planing and rabbeting wood.  They do require a

good drill press with a solid arbor.  The down side is that the

planed wood needs more scraping/sanding then that from a thickness

planer --- but this isn't a problem for a luthier.

--MORE--(84%)Another good thing about safety planers is that they don't have a

depth limit.  You could plane 12" timbers with adequate support and

saintly patience.

--

Douglas S. Rand

Internet:   <dsrand@mitre.org>

Snail:      MITRE, Burlington Road, Bedford, MA

Disclaimer: MITRE might agree with me - then again...

Amateur Radio: KC1KJ

End of article 4307 (of 4320)--what next? [npq] 4308

Article 4308 (11 more) in rec.woodworking:

From: dsr@mitre.org (Douglas S. Rand)

Subject: Re: Thickness Planer Summary

Message-ID: <DSR.91Aug30103530@chickpea.mitre.org>

Date: 30 Aug 91 17:35:30 GMT

References: <1991Aug28.125132.191521@cs.cmu.edu>

<DSR.91Aug28134341@chickpea.mitre.org> <1991Aug29.162855.3782@amd.com>

Sender: news@linus.mitre.org (News Service)

Organization: The MITRE Corporation, Bedford, MA

Lines: 12

In-Reply-To: phil@brahms.amd.com's message of 29 Aug 91 16:28:55 GMT

Nntp-Posting-Host: chickpea.mitre.org

I forgot to mention,  you can get safety planers from most of the

supply houses, Sears, Trendlines, etc.  They also make them for

radial arm saws.  If you buy one,  I recommend to spend the little

extra for the solid aluminum body versions.

--

Douglas S. Rand

Internet:   <dsrand@mitre.org>

Snail:      MITRE, Burlington Road, Bedford, MA

--MORE--(92%)Disclaimer: MITRE might agree with me - then again...

Amateur Radio: KC1KJ

End of article 4308 (of 4320)--what next? [npq] 4316

Article 4316 (4 more) in rec.woodworking:

From: stanj@hpnmdla.sr.hp.com (Stan Jaffe)

Subject: Re: Drill presses (was: Re: Delta Rebate Offer)

Message-ID: <4890165@hpnmdla.sr.hp.com>

Date: 3 Sep 91 16:47:14 GMT

References: <1991Aug28.205634.20117@athena.cs.uga.edu>

Organization: HP Network Measurements Div, Santa Rosa, CA

Lines: 28

In rec.woodworking, hsc@stroid.sybase.com (Howard Cohen) writes:

Other useful accessories:

A mortising attachment. This allows you to drill square holes with

special chisel bits.  You can't use this if you get a bench-top model

drill press (at least with the Delta).

I have a bench top drill press with a mortising attachment.  Perhaps the

requirement of a floor press is only true for the Delta, as you implied.

You may want to make yourself a table for your drill press with a fence.

--MORE--(55%)        I've seen this lots of time, but I don't have one on my drill press.

I rely on my cross vice for practically everything.

I built a table, approximately 1' X 4' that I have mounted in a cross vise on

my drill press table.  I also have an adjustable fence on the table.  The

rough adjustments are made by moving the fence relative to the table.  The

fine adjustments are made using the cross vise.  The threads on the cross vise

are such that one revolution results in 1/8" of table movement.  I'm quite

pleased with this setup.

Stan Jaffe    A.K.A.   stanj@sr.hp.com

HP Santa Rosa, Calif.

"Not responsible for any opinions but my own"

------------------------------------------------------------------

End of article 4316 (of 4320)--what next? [npq]