20/16-bit AGC Architecture

Harvard architecture with a ROM of 32k words of 20 bits for instructions only (6 μsec); a RAM of up to 4k words of 16 bits for data only (6 μsec); 1 parity bit in ROM & RAM.

Central registers of 16 bits with no parity bit: A & L, ALS, Q, Z, X, Y, B, RAP.

Other central registers with no parity bit: IR/14, SF[ixed]/15, SE[rasable]/12.

Input/Output registers or channels TBD.

No program interrupt logic; instead, events and timeouts set bits for RUPT to test.

A – Accumulator for single precision and upper half of double precision.

L – Lower accumulator for double precision, specifically including lower half of a product from MP and MPAD, and lower half of a dividend for DV.

ALS – A&L Shadow, used in shifting A and L together, also as AS and LS.

Q – Quick-turnover register for return address and other very-short-term data.

Only the above central registers are addressable, as 0001, 0002, and 0003.

Z – Instruction location counter, usually contains the (necessarily ROM) address of the next instruction; see also incrementing  property of SF.

X – First latch for augend in any addition except Z increment.

Y – Second latch for addend; pseudo ‘U’ is the sum.

B – Buffer for multiple purposes; pseudo ‘C’ is its ones-complement outputs.

RAP – Return-Address Pointer, for LIFO handling of RAs in low RAM.

IR – Interrupt Requests, prioritized array of bits, one per interrupt type.

SF – Select Fixed: holds address to drive ROM; incremented content is readable..

SE – Select Erasable: holds address to drive RAM.

General Description

Instructions of 19 bits plus odd parity occupy up to 32K of ROM (aka Fixed). Data words of 15 bits plus odd parity occupy up to 4K words of RAM (aka Erasable). A few central registers (A, L, Q) have low-number addresses to be addressable like RAM. Some low-number RAM locations are special in the sense that their content is modified by external events, most often incrementing or decrementing to track spacecraft sensor values. There are no special-register locations that perform shifting or other data editing. Whenever possible, a Memory Cycle Time (MCT) performs RAM and ROM cycles simultaneously.

There are 8 instructions that have 15-bit address fields, of which two are special cases because their 15-bit field is constant data which one emits into the A register, and the other substitutes it for whatever data is next read from erasable. The others use their 15-bit fields to address ROM (only) for transfer-of-control purposes.

Of the other instructions, none can address ROM. They all. Those that need to address data in RAM devote 12 bits to the purpose, and include a 13th bit to indicate indirect addressing. When they perform indirect addressing, the final address is taken from the rightmost 12 bit positions, ignoring the leftmost 3 bits which can be used for loop counting. There is one exceptional case of indirect addressing, PRET*, which has no direct-addressing form, and uses the indirect address to access a word of interpretive code in ROM. There is an INDEX instruction that adds part or all of its 15-bit operand to the 19-bit next instruction, not affecting any bits above the address field defined for that instruction type. Instructions addressing A, L, or Q access RAM when appropriate[?].

A comprehensive set of shifting instructions, all double precision, support the native add and subtract instructions in both precisions. Multiplication uses single-precision inputs to create a double precision product, and has an option to include the addition of a single-precision number aligned with the lower half of the product. Division uses a double-precision dividend and a single-precision divisor, yielding a single-precision quotient and a single-precision remainder.

Input and output instructions address numbered channels and can perform Boolean functions to read or write only selected bits in any channel. Also, a variety of I/O operations work by stealing an individual RAM cycle, usually to increment or decrement a data word in RAM.

There are a number of 19-bit codes corresponding to miscellaneous operations such as absolute value, normalizing shift, square root, and sine-cosine. An important example is REVIF, which reverses the sense of the next IF type. Of these, as many as possible are implemented within the budget of logic hardware; the remainder call emulation routines. Having no address field, these are unaffected by INDEX.

There is no program interrupt in the customary sense. Instead, any timing out or other event that can change the priority of a job sets a flag bit in an Interrupt Request register, and at convenient but frequent moments the program is required to perform the no-address RUPT instruction (take highest priority interRUPT). If it finds any non-zero bits, it puts its own location into Q and transfers control to ROM location 00001. Code in the “time vicinity” of a Copy Block (CPBK) instruction need not have RUPTs because CPBK performs that function after transferring each word. The hardware restart GOJAM clears all central registers, transferring control to 00000, which must contain a GOTO.

The go-to and call functions are performed by the same operation code, documented as CALL to gratify Dijkstra fans, even though the assembler accepts GOTO or CALL. Any of the five IF instructions also functions as CALL if its branching condition is satisfied, copying Z into Q to serve as a return address. There is an EXEC instruction that gives control to the addressed instruction without changing Z, so that any instruction that doesn’t arbitrarily change Z performs its function and uses the unchanged Z to pass control back to whatever follows the EXEC. The exceptions are CALL and any IF-type instruction that actually branches. A special execute type, PRET*, prepares its ROM operand for processing as an interpretive instruction by distributing its 19 bits to A and L.

Parts of the design are motivated by the need, in abnormal circumstances, to insert into RAM something that will act like a program even though it can’t quite look like one. Ground controllers use a piece of support software that breaks each 19-bit instruction created on the ground into two 15-bit words to insert into available RAM space. They then direct the computer to a program that uses these parameters to effectively interpret the machine’s own native instructions by executing a standard copy of each desired instruction that generally uses indirect addressing to access the desired location.

Instruction Formats in ROM

Immediate data and instruction reference types (branches):

D...D is immediate data; A…A is address, ROM only. Both types are indexable.

Octl Op Code 19 18 17 16 15 14 13 12 11 10 09 08 07 06 05 04 03 02 01

1 0  DATA=   1  0  0  0  D  D  D  D  D  D  D  D  D  D  D  D  D  D  D

1 1  DATAA   1  0  0  1  D  D  D  D  D  D  D  D  D  D  D  D  D  D  D

1 2  EXEC    1  0  1  0  A  A  A  A  A  A  A  A  A  A  A  A  A  A  A

1 3  CALL    1  0  1  1  A  A  A  A  A  A  A  A  A  A  A  A  A  A  A

1 4  IFPNZ   1  1  0  0  A  A  A  A  A  A  A  A  A  A  A  A  A  A  A

1 5  IFNZ    1  1  0  1  A  A  A  A  A  A  A  A  A  A  A  A  A  A  A

1 6  IFNEG   1  1  1  0  A  A  A  A  A  A  A  A  A  A  A  A  A  A  A

1 7  IFNOV   1  1  1  1  A  A  A  A  A  A  A  A  A  A  A  A  A  A  A

Data handling operations that use RAM addresses for direct or indirect (*):

Op Code 19 18 17 16 15 14 13 12 11 10 09 08 07 06 05 04 03 02 01

(gen’l) 0  C  C  C  C  C  *  A  A  A  A  A  A  A  A  A  A  A  A

0 00 – 0 06  LDA & LDL & DLD & LDQ   4  4

0 10 – 0 16  STA & STL & DST & STQ   4  8

0 20 – 0 26  XCA & XCL & DXC & XCQ   4 12

0 30 – 0 36  T   & DT  & CP  & DCP   4 16

0 40 – 0 46  AD  & DAD & SU  & DSU   4 20

0 50 – 0 56  MP  & MSK & DV  & MPAD  4 24

0 60 – 0 62  LAN & DLN               2 26

0 64 – 0 70  INC & RED & NDX         3 29

0 72 – 0 73  STCO(direct) & PRET*    1 30

0 74 – 0 75  shift & inout (no *)    1 31

0 76 – 0 77  miscellaneous (no *)    1 32

The LD/ST/XC instructions have special modes when directly addressing the central registers they serve: LD→T [Test], ST→Z [clear to Zero], XC→C [Complement], e.g. TA = LDA A, ZL = STL L, CQ = XCQ Q.

Indirect address used by all but the last 3 lines of this type; LLL is a loop counter (1-8):

15 14 13 12 11 10 09 08 07 06 05 04 03 02 01

L  L  L  E  E  E  E  E  E  E  E  E  E  E  E [Addresses RAM only]

Loop counter LLL is used only by RED, which decrements E…E by 1 or 2 per last op and tests LLL for 000, setting condition code; decrements LLL by 1 if it was not 000. PRET* is always indirect and always accesses ROM through its indirect address.

Op Code 19 18 17 16 15 14 13 12 11 10 09 08 07 06 05 04 03 02 01

(shift) 0  1  1  1  1  0  0  T  T  T  D  S  S  S  N  N  N  N  N

All shifts affect both A and L, but TTT=Type may clear either A or L initially; DSSSNNNNN=twos-complement count with extended sign bit to determine direction: positive=left, negative=right. INDEX affects bits 09-01 only.

Op Code 19 18 17 16 15 14 13 12 11 10 09 08 07 06 05 04 03 02 01

(inout) 0  1  1  1  1  0  1  D  B  B  C  C  C  C  C  C  C  C  C

D=direction, BB=Boolean function, C…C=channel. INDEX affects bits 09-01 only.

Op Code 19 18 17 16 15 14 13 12 11 10 09 08 07 06 05 04 03 02 01

(misc)  0  1  1  1  1  1  E  C  C  C  C  C  C  C  C  C  C  C  C

Miscellaneous: RUPT, SQRT, etc. E=Emulated rather than native. C…C=detail code or call address in low 4k of ROM.

Brief Explanation of Instruction Operation Codes

Immediate data and instruction reference (branching); none of these fetches DATA=

DATA=	General immediate DATA, for use with any instruction that fetches data.

Bits 15-01 replace whatever is next read from RAM, on a one-shot basis.

DATAA	Default case for loading immediate DATA into A.

EXEC		EXECutes addressed instruction without transferring control.

CALL		CALL or GOTO, unconditional; target decides which (also true of IFs).

IFPNZ		IF condition code shows Positive Non-Zero, branch.

IFNZ		IF condition code shows Non-Zero, branch.

IFNEG		IF condition code shows NEGative (necessarily non-zero), branch.

IFNOV	IF condition code shows NO oVerflow, branch.

The sense of any IF is reversed if it immediately follows a REVIF instruction:

IFN|Z		IF condition code shows Negative or Zero, branch.

IFZER		IF condition code shows ZERo, branch.

IFP|Z		IF condition code shows Positive or Zero, branch.

IFOVF		IF condition code shows OVerFlow, branch.

Operations that use erasable (RAM) for fetching and/or storing:

LDA		LoaD A (LDA A = TA: Test A for sign, zeroness, overflow)

LDL		LoaD L (LDL L = TL: Test L for zeroness only)

DLD		Double LoaD into A and L (DLD L = TD: Test Double c(A,L)

LDQ		LoaD Q (LDQ Q = TQ: Test Q for sign, zeroness, overflow)

STA		STore A (STA A = ZA: Zero A)

STL		STore L (STL L = ZL: Zero L)

DST		Double Store from A and L (DST L = ZD: Zero Double c(A,L)

STQ		STore Q (STQ Q = ZQ: Zero Q)

XCA		eXChange A (XCA A = CA: Complement A)

XCL		eXChange L (XCL L = CL: Complement L)

DXC		Double eXChange A and L (DXC L = CD: Complement A,L)

XCQ		eXChange Q (XCQ Q = CQ: Complement Q)

T		Test number in erasable/RAM

DT		Double precision Test number in erasable/RAM

CP		ComPare A with number in erasable/RAM (CC as if c(A) – data word)

DCP		Double ComPare c(A,L) with DP number in erasable/RAM)

AD		ADd to A

DAD		Double precision ADd to A,L

SU		SUbtract from A

DSU		Double precision SUbtract from A,L

MP		MultiPly c(A) (DP product in A,L)

MSK		apply MaSK to A (logical AND)

DV		DiVide c(A,L) (quotient in A, remainder in L)

MPAD		MultiPly c(A) and ADd b(L) (DP product in A,L)

LAN		Load A Negative (LAN A = NA: Negate c(A))

DLN		Double precision Load Negative (DLN L = Negate c(A,L))

INC		INCrement by 1 or 2 per most recent precision

RED		REDuce by 1 or 2 as above; decrement & test loop count field

NDX		iNDeX next instruction by adding to its address field

STCO		STore Carry Out: 0, +1, or -1; copy A16 to A15 (no *)

PRET*		distribute interPRETive instruction from ROM into A and L; 
		post-increments the indirect address it used to access ROM

Input/Output operations addressing up to 512 channels; IR is 000; L is 002; Q is 003

READ		Read into A

ROR		Read & OR with A

RAND		Read & AND with A

RXOR		Read and XOR with A

WRITE	Write from A

WOR		Write & OR with channel

WAND	Write & AND with channel

WXOR	Write & XOR with channel

Shifts in A,L or Q (NDXing shift count may reverse shift direction)

SDL		Shift Double precision Left (signed)

SDR		Shift Double precision Right (signed)

SDRR		Shift Double precision Right (signed) and Round

SDLU		Shift Double precision Left Unsigned

SDRU		Shift Double precision Right Unsigned

SAL		Shift A Left (signed)

SAR		Shift A Right (signed)

SARR		Shift A Right (signed) and Round

SALU		Shift A Left Unsigned

SARU		Shift A Right Unsigned

CYCA		Cycle A [QUESTION SIGNED SHIFT COUNT]

SLL		Shift L Left			(L is inherently unsigned)

SLR		Shift L Right			(L is inherently unsigned)

SLRR		Shift L Right and Round	(L is inherently unsigned)

Miscellaneous operations with no address

NOP		No OPeration, ocasionally useful in indexed EXECs

PRA		Push Return Address onto return address list (converts to call type)

RUPT		Take all requested interRUPTs before proceeding

RSM		ReSuMe when interrupt is finished; restore saved CC

SAVCC	SAVe Condition Code

RSTCC	ReSTore Condition Code

REVIF		Edit condition code to REVerse sense of next IF

NORM	NORMalize c(A) and put shift count in Q

DNORM	Double precision NORMalize c(A,L) and put shift count in Q

ABA		ABsolute value of b(A) to A

DAB		ABsolute value in b(A,L) to A,L

ABQ		ABsolute value of b(Q) to Q

ANGLE	Resolves false overflow of an angle by copying A15 to A16

R		Round c(A) per high bit of c(L)

DR		Double precision Round c(A,L) per low bit of c(L)

SQRT		SQuare Root of c(A,L), root to A, remainder to L

SC		Sine of b(A) to A, Cos of b(A) to L

CPBK		CoPy BlocK of words (from-address in A, to-address in L, count in Q,

able to accommodate interrupts after any word copy). When preceded

by DATA=, copies the immediate data to the whole destination block.

Note on Condition Codes

In principle, the condition code can be represented in two bits, one for the sign and one for nonzero-ness, with a special case for overflow, which confers indeterminacy on those two conditions. But to accommodate the REVIF feature, it is actually kept as four bits, one to define the correct response by each of the IFs. REVIF translates the four-bit code to a different four-bit code to define the correct response under the reversed-if rule.

Cond	PNZ	NZ	NEG	NOV	Code/reversed N|Z	ZER	P|Z	OV

= 0	no	no	no	yes	0001/1110	yes	yes	yes	no

> 0	yes	yes	no	yes	1101/0010	no	no	yes	no

OVF	no	no	no	no	0000/0001	no	no	no	yes

< 0	no	yes	yes	yes	0111/1000	yes	no	no	no

The execution of any of these IF types notices the reversed state and if necessary translates back to a normal code. Also, while any reversed code is in force, no updates of it can be made, so any REVIF should by followed closely by the intended IF, either immediately or with at most an INDEX intervening. The code, in either state, can be saved in a dedicated special register by SAVCC and restored by RSTCC; this is done automatically when entering and leaving interrupt code (tasks).

Timing of the IF types depends on both the IF/ELSE decision. The decision is taken in the first pulse time: in the ELSE case, the one MCT fetches the next instruction per Z.

Note on Double Precision

The sign and 29 value bits of double precision numbers appear in A,L joined together for shifting. The one sign bit occupies bit A16 and (unless overflow is present) A15 as well. Bits A14-A01 hold the upper 14 bits of the value. Bits L16-L02 hold the lower 15 bits of the value, implying a left shift when L is loaded from memory and a right shift when stored. Bit L01 is normally zero, but after right shifts it contains a bit usable for rounding by DR (Double precision Round).

The following discussion of how shifting uses and affects A’s two sign bits is simplified by not mentioning ALS, the shadow A and L. Left shifts always zero L01.

Unsigned right shifts: each shift copies A15 into A14 and zeros both A16 and A15.

Unsigned left shifts: each shift copies A14 into both A15 and A16, no overflow check.

Signed right shifts: each shift copies A16 into A15, b(A15) into A14, and so on.

Signed left shifts, overflow present: each shift leaves A16 and A15 unchanged.

Signed left shifts, overflow absent: each shift leaves A16 as is, copies A14 into A15.

This design implements a policy to allow a one-level overflow to be “cured” by a signed right shift, just accepting that higher-level overflows won’t recover the information they have lost. This is consistent with the ability of another addition or subtraction to resolve a temporary one-level overflow without comment. Another policy is to latch any overflow caused by signed left shifting, so that IFNOV can respond to it. As in Block II, storing A (with overflow present or absent) copies A16 to the sign-bit position in memory and doesn’t copy A15 anywhere, but in Block III does not skip on overflow. Signed right shifts also convert a negative result to all-zero when all the significant digits are shifted off the right end.

Single-precision Round uses both the sign in A16 and all of c(L) to determine the action. If positive, a one in L16 increments c(A). If negative, the XOR of L16 and nonzeroness of L15-L01 decrements (A). Either change in A could produce overflow; if overflow was present to begin with, the change could in rare cases change the sign of the overflow but not its presence—those cases are 077777 and 100000. In all cases, Round clears L afterwards, since c(A,L) may be no longer a valid double-precision number.

Double-precision Round uses the sign in A16 and c(L01) to determine the action. If positive, a copy of c(L01), aligned with L02, is added in double precision. If negative, a copy of c(L01), aligned with L02, is subtracted in double precision (by adding a string of 31 ones to A16-L02 when L01 contains a one). The possibilities for creating or changing overflow are similar to those of Round. DR always clears L01 when done.

Note on Multiplication

In order to support double-precision (DP) multiplication, MP multiplies a 15-bit signed multiplicand in A by a 14-bit signed multiplier from memory, and delivers the 29-bit signed product to A16-L02. In most cases, the multiplicand will have only 14 significant bits, with A16 = A15 as usual, and the shift-and-add operations will proceed much as in Block II except for L16-L15 being value bits instead of sign bits. But when the multiplicand is the lower part of a DP number, as is required in DP multiplication, its 15 value bits have to occupy A15-A01, looking like but not actually being an overflow state. In principle, the ordinary way to achieve this condition would be to start with a DP number in A,L and do a signed left shift of 15, ready for MP to copy the 16-bit result from A to B. In practice, a special shift instruction SL15 emulates that 15-bit shift in one pulse time, by copying L16-L02 into A15-A01. If all 15 of the copied bits are zeros, A16 is alos set to zero. MP starts by copying all of A to B. If MPAD rather than MP is being done, the addend in Q is copied to A, otherwise A is zeroed. Note: overflow in c(Q) will destroy the product if adding the multiplicand causes second-level overflow. See example below for how to prevent this problem.

As in Block II, MP examines the multiplier fetched from memory; if it’s negative, both it  and the multiplicand in B are negated. Then the the multiplier (now in its absolute-value form) goes to L to control the ZIP-ZAP cycles for multiplication. The product sign is also preserved in what amounts to a 17th bit, so that when the adder output is shifted right 2 bit positions into A14-A01 and L16-L15, it is copied into both A16 and A15. This logic still needs careful simulation of the pulses in ZIP and ZAP.

Sample Subroutine, Oriented More to Calculation than to Logic

P01 Double precision multiply, MPAC *= c(A,L), keeping acceptable

R02 double precision but no third word of product.

03 01234              DMP      PRA              CALL, not GOTO

04 01235                       DST    DOperand

05 01236                       SL15             Orig L = multiplicand

06 01237                       MP     MPAC      Top half = multiplier

07 01240                       DST    DTemp     First cross product

08 01241                       DLD    MPAC      Addr assembles w/ +1

09 01242                       SL15             Bottom = multiplicand

10 01243                       MP     DOperand  Second cross product

11 01244                       DAD    DTemp     Sum of cross products

12 01245                       STCO   DTemp     Carry-out for top half

13 01246                       STA    Q         Addend won’t incl OV

14 01247                       LDA    MPAC

15 01250                       MPAD   DOperand  Top * top + Q addend

16 01251                       AD     DTemp     Carryout (cross prods)

17 01252                       DST    MPAC      Product in A,L also

18 01253                       RTURN            Assembles as GOTO 0

R19 To keep triple precision (though unrounded), multiply the lower

R20 halves, somehow working around the 14-bit multiplier restriction.

R21 Shift this signed product right to make a DP number to add to the

R22 first cross product in DTemp, which I believe will yield a sum

R23 free of overflow. Don’t attempt an MPAD here. Not essential to do?

P50 Run code from erasable memory, using 2 words (op code and address)

R51 for each instruction. Emulates the native instructions without

R52 giving up control except where addressing high ROM requires it.

R53 This “native-interpreter” (NI) task, whose priority is normally

R54 zero, doesn’t run at all except when ground controllers load the

R55 code into RAM and then trigger a dedicated interrupt to change

R56 the task’s priority. Part of the ground load is the RAM address

R57 of the first instruction to be emulated, in location NISF.

R58 Note: the condition code (CC) and its saved backup are such

R59 indeterminate as the NI code begins. RUPT will not appear in

R60 programs, being unnecessary since the NI itself performs RUPT

R61 frequently. Hence, no NI emulation of RUPT.

RunFromE STQ    NIsaveQ   *RunFrmE is a permanent

LDQ*   NISF

STQ    NIopCode  Point to NI instruction

INC    NISF      Point to address word

LDQ*   NISF

STQ    NIaddr    Usable as a direct address

LDQ*   NIaddr    Don’t know if legit, but:

STQ    NIindir    could be indirect address

DATAQ  SignBit   Octal 40000

XCQ    NISFback  Set unINDEXed for next inst

IFNEG  +2        Skip restore when unINDEXed

STQ    NISF      Restore saved NISF post NDX

+2 LDQ*   NIopCode

IFNEG  TransCtl  Any of 6 TC types: TransCtl

DATAQ  IndirMsk  Octal 10000 [UPD FOR IPRET]

MSK    NIaddr    Miscs always look direct ! Q NOT A !

REVIF

IFZER  Emulate

Indirect LDQ    NIindir

STQ    NIaddr    Emulate indirect addressing

GOTO   Emulate

TransCtl DATAQ  FonlyMsk  Octal 70000

MSK    NIaddr    Get F-vs-E field, bits 15-13

IFNZ	  MakeDir   F => direct in native mode

DATAQ  FonlyMsk  Octal 70000

MSK    NIindir   Get F-vs-E field, bits 15-13

IFNZ   Emulate   F => indirect in native mode

INC    NIopCode

INC    NIopCode  NI modes follow native by +2

GOTO   Emulate

MakeDir  LDQ    NIaddr    In case INDEX changed E to F

STQ    NIindir

Emulate  INC    NISF

LDQ    NISF

STQ    NIZ       Both point to next opcode wd

LDQ    NIsaveQ

RSTCC            Note: overflow is a CC state

EXEC*  NIopCode

SAVCC

STQ    NIsaveQ   Because RUPT is a CALL if go BUT...

NInext   IFIW   NIRupts   *NInext is perm, for TransCtl

LDQ    NIsaveQ   In case another job had a go

GOTO*  *RunFrmE  Until a native TC type escapes

P   Emulation routines for 15-bit address types. Note that all of them

R   except DATA= and DATAQ (which are after all just data operations)

R   begin with

R   a 15-bit address type providing a “sign” bit for the interpreter to

R   know which emulated codes can do transfers of control. The TC types

R   were separated into direct and indirect by the special assembler

R   mode used by ground control. For each TC type, a one-word native

R   mode implementation is followed immediately by a one-word way to

R   resume erasable operation, then its NI mode logic.

DATAQ    LDQ    NIaddr    No NI mode logic required

+DATA    AD     NIaddr    No NI mode logic required

CALL     CALL*  NIaddr

GOTO*  *RunFrmE  In case of return from F

NICALL   CALL   +1        Convert EXEC to CALL

PRA              Sets up return to EXEC +1

LDQ    NIaddr

XCQ    NIZ

STQ    NIsaveQ

LDQ    NIZ

STQ    NISF

INC    NIZ

GOTO*  *NInext

CALL*    CALL*  NIindir   INDEX may set = NIaddr

GOTO*  *RunFrmE  In case of return from F

NICALL*  CALL   +1        Convert EXEC to CALL

PRA              Sets up return to EXEC +1

LDQ    NIindir

XCQ    NIZ

STQ    NIsaveQ

LDQ    NIZ

STQ    NISF

INC    NIZ

GOTO*  *NInext

EXEC     EXEC*  NIaddr

GOTO*  *RunFrmE  In case of return from F

NIEXEC   CALL   +1        Convert EXEC to CALL

PRA              Sets up return to EXEC +1

LDQ    NIaddr

STQ    NISF      Notice no change in NIZ

GOTO*  *NInext

EXEC*    EXEC*  NIindir

GOTO*  *RunFrmE  In case of return from F

NIEXEC*  CALL   +1        Convert EXEC to CALL

PRA              Sets up return to EXEC +1

LDQ    NIindir

STQ    NISF      Notice no change in NIZ

GOTO*  *NInext

IFPNZ    IFPNZ* NIaddr

GOTO*  *RunFrmE  In case of return from F

NIIFPNZ  IFPNZ* *NICAL1   Permanent -> NICALL+1

IFPNZ*   IFPNZ* NIindir

GOTO*  *RunFrmE  In case of return from F

NIIFPNZ* IFPNZ* *NICAL*1  Permanent -> NICALL*+1

IFNZ     IFNZ*  NIaddr

GOTO*  *RunFrmE  In case of return from F

NIIFNZ   IFNZ* *NICAL1    Permanent -> NICALL+1

IFNZ*    IFNZ*  NIindir

GOTO*  *RunFrmE  In case of return from F

NIIFNZ*  IFNZ* *NICAL*1   Permanent -> NICALL*+1

IFNEG    IFNEG* NIaddr

GOTO*  *RunFrmE  In case of return from F

NIIFNEG  IFNEG* *NICAL1   Permanent -> NICALL+1

IFNEG*   IFNEG* NIindir

GOTO*  *RunFrmE  In case of return from F

NIIFNEG* IFNEG* *NICAL*1  Permanent -> NICALL*+1

IFNOV    IFNOV* NIaddr

GOTO*  *RunFrmE  In case of return from F

NIIFNOV  IFNOV* *NICAL1   Permanent -> NICALL+1

IFNOV*   IFNOV* NIindir

GOTO*  *RunFrmE  In case of return from F

NIIFNOV* IFNOV* *NICAL*1  Permanent -> NICALL*+1

P   Emulation routines for 12-bit address types. Note that all of them

R   begin with a non-15-bit address type providing a “sign” bit for the

R   interpreter to know which emulated codes cannot do transfers of

R   control. All are one-word native-mode implementations except INDEX.

LDA      LDA*   NIaddr    Load A, set CC, REDL=1, OV=

LDL      LDL*   NIaddr    Load L, set CC, REDL=1, OV=

DLD      DLD*   NIaddr    Load A&L, set CC, REDL=2, OV=

LDQ      LDQ*   NIaddr    Load Q, set CC, REDL=1, OV=

STA      STA*   NIaddr    Store A, set CC, REDL=1, OV=

STL      STL*   NIaddr    Store L, set CC, REDL=1, OV=

DST      DST*   NIaddr    Store A&L, set CC, REDL=2, OV=

STQ      STQ*   NIaddr    Store Q, set CC, REDL=1, OV=

XCA      XCA*   NIaddr    Exchange A, set CC, REDL=1, OV=

XCL      XCL*   NIaddr    Exchange L, set CC, REDL=1, OV=

DXC      DXC*   NIaddr    Exchange A&L, set CC, REDL=2, OV=

XCQ      XCQ*   NIaddr    Exchange Q, set CC, REDL=1, OV=

LAN      LAN*   NIaddr    Load A negative, set CC, REDL=1, OV=

DLD      DLD*   NIaddr    Load A&L negative, set CC, REDL=2, OV=

CP       CP*    NIaddr    Compare single, set CC, REDL=1, OV=

DCP      DCP*   NIaddr    Compare double, set CC, REDL=2, OV=

AD       AD*    NIaddr    Add single, set CC, REDL=1, OV per

DAD      DAD*   NIaddr    Add double, set CC, REDL=2, OV per

SB       SB*    NIaddr    Subtract single, set CC, REDL=1, OV per

DSB      DSB*   NIaddr    Subtract double, set CC, REDL=2, OV per

MP       MP*    NIaddr    Multiply, set CC, REDL=1, OV=

MPAD     MPAD*  NIaddr    Multiply & add, set CC, REDL=2, OV=

DV       DV*    NIaddr    Divide, set CC, REDL=1, OV per

ORQ      ORQ*   NIaddr    Or, set CC, REDL=1, OV=

ANDQ     ANDQ*  NIaddr    And, set CC, REDL=1, OV=

XORQ     XORQ*  NIaddr    Exclusive Or, set CC, REDL=1, OV=

INC      INC*   NIaddr    Increment, set CC, REDL=1, OV per

RED      RED*   NIaddr    Reduce per REDL, set CC, REDL=, OV per

IPRET*   IPRET* NIindir   Load ROM word into A,L for interpreting

INDEX    CALL   +1        Use of Z requires special NI logic

PRA              Sets up return to EXEC +1

DST    NIsaveAL  DP work weird but efficient

LDL*   NIaddr    Amount to be added by INDEX

DAD*   NIZ       L holds addr result ready to mask

LDA*   NIZ       NIopCode-to-be

STA    NIopNdxd

DATAQ  LDA

CP     Q

IFNEG  Index15b  NI slot < LDA => DATAQ, DATA=, & TC types

DATAQ  SHIFTS

CP     Q

IFNEG  Index12b  NI slot < SHIFTS => RAM addresses

DATAQ  MISCELS

CP     Q         NI slot < MISCELS => shifts & I/O

REVIF

IFP|Z  IndxDone  Indexing has no effect on MISCELS

Index09b DATAQ  SHIOMsk   Octal 00777 for shifts and I/O

GOTO   TrimIndx

Index12b DATAQ  EMemMsk   Octal 07777 for RAM addresses

GOTO   TrimIndx

Index15b DATAQ  AllBits   Octal 77777 for ROM addresses

TrimIndx STQ    AddrMsk

ANDQ   L         Trim sum after index amt added

XCQ    AddrMsk

CQ               Mask for the part that stays put

DLD*   NIZ       Address before indexing to L

ANDQ   L

ORQ    AddrMsk   Put whole address together

STQ    NIadNdxd

DATAQ  NIopNdxd

XCQ    NISF

STQ    NISFback

IndxDone DLD    NIsaveAL

RSTCC

GOTO*  *NInext

Support for Interpretive Instructions in ROM

The guiding principle for the Block III Interpreter is to have at least the same functional capabilities as the Block II Interpreter but with different instruction formats and the wider access to memory implied by the Block III architecture. The high-level interpretive instructions, 19 bits in length like the native instructions, have the following general formats, effective only as operands of a native PRET* instruction:

19 18 17 16 15 14 13 12 11 10 09 08 07 06 05 04 03 02 01 [“OPS”]

0  X  X  X  X  X  X  Y  Y  Y  Y  Y  Y  Z  Z  Z  Z  Z  Z

X...X=1st operation code, Y...Y=2nd operation code, Z...Z=3rd operation code;
but in some cases (e.g. shifting) a 6-bit field is a parameter for the prior operation

19 18 17 16 15 14 13 12 11 10 09 08 07 06 05 04 03 02 01 [“A15”]

1  0  X  X  A  A  A  A  A  A  A  A  A  A  A  A  A  A  A

XX=00 if unindexed; 01 or 10 = apply index register 1 or 2; A…A= Address;
if for data, an address > 4K points to a DATA= to supply data from fixed/ROM;
if for transfer of control, an address < 4K performs indirect addressing to any level

19 18 17 16 15 14 13 12 11 10 09 08 07 06 05 04 03 02 01 [“A12”]

1  0  X  X  *  0  0  A  A  A  A  A  A  A  A  A  A  A  A

XX=00 if unindexed; 01 or 10 = apply index register 1 or 2; * = indirect (one level);
A…A= Address (RAM); usable with general shifts to indirectly address the shift count

19 18 17 16 15 14 13 12 11 10 09 08 07 06 05 04 03 02 01 [“GSH”]

1  0  X  X  S  S  S  S  S  S  S  S  N  N  N  N  N  N  N

XX=00 if unindexed, 01 or 10 = apply index register 1 or 2;
S...S=extended Sign; N...N=base shift count (2’s complement), for general shifts

19 18 17 16 15 14 13 12 11 10 09 08 07 06 05 04 03 02 01 [“IMD”]

1  0  1  1  D  D  D  D  D  D  D  D  D  D  D  D  D  D  D

D…D = immediate Data for certain operations that read single precision data

19 18 17 16 15 14 13 12 11 10 09 08 07 06 05 04 03 02 01 [“STO”]

1  1  X  X  *  S  S  A  A  A  A  A  A  A  A  A  A  A  A

XX=00 if unindexed, 01 or 10 = apply index register 1 or 2; * = indirect (one level);
SS = 00 for STORE, 01 for STODL, 10 for STOVL, 11 for STCALL;
A…A= Address (RAM);

Transition from native to interpretive mode is performed by a call to the interpreter, followed by interpretive instructions as in the Block II interpreter except for their being in any of the formats given above. The interpreter software saves the return address in ZPret and performs PRET* ZPret to copy an interpretive program word into AL in two different ways (unoccupied bits in AL are set to zero), and increments ZPret.

Bit 19 = 0: bits 18-13 into A06-01, bits 12-01 into L16-05, Condition Code (CC) = PNZ;

Bit 19-18 = 10: bits 17-16 into A02-01, bits 15-01 into L16-02, CC = ZER.

Bit 19-18 = 11: bits 17-16 into A02-01, bits 15-01 into L16-02, CC = NEG.

It increments ZPret and uses the AL data to invoke the appropriate routines to perform the interpretive program word’s function, whether as operation codes or an address. The assembler will insist on the first word after CALL Intrpret being an OPS type (positive).

Intrpret STQ    ZPret     Normal entry – don’t put RA in stack

NextOPS	 PRET*  ZPret     First op to A, other two to L, ZPret++

IFMN	  TypeSTO	If negative here, must be type STO

AnyOp	 STL    OtherOps	Two more ops, or one, or none

NDX	  A

EXEC	  Dispatch	Every Op routine exits to NextOp

NextOp	 LDL	  OtherOps

SLU	  6

TA

IFNZ	  AnyOp

GOTO	  NextOPS

Dispatch NOP			Not usual, but still possible

+01	 CALL	  ReversIf	Set flag and skip RUPT

+02	 CALL	  Shift	Capture 6-bit parameter to distinguish

+03	 CALL	  Flags	Capture 6-bit parameter to distinguish

+04	 CALL	  Indx	Analogous to Native Mode NDX

..........................

+76	 CALL	  Return

+77	 GOTO	  ZPret	EXIT from interpreter

TypeSTO	 CALL	  ParseAdr	Common address field parser

T	  MPACMode

..........................

GOTO	  NextOPS

Sample Interpretive Routine

P01 Vector dot product, MPAC * memory, rounding result and clearing

R02 MPAC +2. Finds big-end vector address in OperAddr.

VDotProd DLD*   OperAddr	// Auto-increments indirect address

CALL   DMP

DST    Tempac	// Assembled as DST Tempac+1

ZA

NDX	  VAC

LDL    MPAC +2

DST    Tempac +2 // Initialize temp accumulator

NDX	  VAC

DLD    MPAC +3

NDX	  VAC

DST    MPAC

DLD*   OperAddr	// Auto-increments indirect address

CALL   DMP

DAD    Tempac

DST    Tempac

ZA

NDX	  VAC

LDL    MPAC +2

DAD    Tempac +2

DST    Tempac +2 // Build rounding quantity

NDX	  VAC

DLD    MPAC +5

NDX	  VAC

DST    MPAC

DLD*   OperAddr	// Auto-increments indirect address

CALL   DMP

DAD    Tempac

NDX	  VAC

DST    MPAC

DZ

NDX	  VAC

XCL    MPAC +2   // Clear MPAC +2 as specified

DAD    Tempac +2 // Complete rounding quantity in A

STA    L

ZA

NDX	  VAC

DAD    MPAC

NDX	  VAC

DST    MPAC      // Round

GOTO   PostPret

Survey of Interpretive Instructions

Operation codes—note: OK to have same names as native ops; assembler will understand. Address type(s) in <angle brackets>.

00 NOP	No OPeration, used to fill out unneeded 6-bit slots	<>

01 REVIF	REVerse sense of next IF; see IF codes in [brackets]	<>

02 SHIFT	shift, precision per Mode, type/counts per parm; if G:<GSH>

03 FLAGS	manipulate/test flagbits: op in 1st parm, number in 2nd <>

04 INDX	Interpretive NDX, adds operand to next address word	<A12>

05 NORM     NORMalize and store count per address			<A12>

06 SETPD	SET PushDown pointer PUSHLOC to RAM address		<A12>

07 COPY	Copy word(s) from RAM to RAM, count in parm	<IMD>	<A12>

10 IFPL	branch IF PLus					[IFMN]	<A15>

11 IFZE	branch IF ZEro					[IFNZ] 	<A15>

12 IFMN	branch IF MiNus	[not strictly necessary][IFPL] 	<A15>

13 IFHIZ	branch IF HIgh Zero				[IFNHZ] 	<A15>

14 IFOV	branch IF OVerflow				[IFNOV] 	<A15>

15 IFOVN	branch IF OVerflow to Native-mode routine	[IFNVN] 	<A15>

16 IFMX1	branch IF IndeX 1 loop not completed	[IFD1] 	<A15>

17 IFMX2	branch IF IndeX 2 loop not completed	[IFD2] 	<A15>

20 LXA1	Load indeX register 1 with addressed or immed data	<A12>

21 LXA2	Load indeX register 2 with addressed or immed data	<A12>

22 LXN1	Load indeX register 1 with Neg of addr or immed data	<A12>

23 LXN2	Load indeX register 2 with Neg of addr or immed data	<A12>

24 XAD1	indeX register 1: ADd addressed or immed data		<A12>

25 XAD2	indeX register 2: ADd addressed or immed data		<A12>

26 XSU1	indeX register 1: SUbtract addressed or immed data	<A12>

27 XSU2	indeX register 2: SUbtract addressed or immed data	<A12>

30 STX	STore indeX register in RAM, address identifies index	<A12>

31 XCHX	eXCHange indeX register in RAM, address IDs index	<A12>

32 SLOAD	Single precision LOAD addressed or immediate data	<A12>

33 TLOAD	Triple precision LOAD addressed data			<A12>

34

35

36

37

40 DLOAD	Double precision LOAD, set Mode = D				<A12>

41 VLOAD	Vector precision LOAD, set Mode = V				<A12>

42 DAD & TAD & VAD	ADd (precision D or T or V per Mode)	<A12>

43 DSU & TSU & VSU	SUbtract (precision D or T or V per Mode) <A12>

44 BDSU & BVSU	Backwards SUbtract (precision D or V per Mode) 	<A12>

45 DMP & DOT	Double precision MultiPly (D) or Vector dot prod<A12>

46 DMPR & VXV	DMP and Round (D) or Vector cross product		<A12>

47 SIGN		negate MPAC if SIGN in memory is negative		<A12>

50 DDV & VXM	Double precision DiVide (D) or Vector x Matrix	<A12>

51 BDDV & MXV	Backwards DDV (D) or Matrix x Vector		<A12>

52 ROUND & VPROJ	ROUND if D Mode, or Vector PROJect if V Mode	<A12>

53 VXSC		Vector mult by SCalar (which is which per Mode)	<A12>

54 V/SC		Vector div by SCalar (which is which per Mode) 	<A12>

55

56

57

60

61 ABS & ABVAL	ABs value of DP/TP scalar (D) or vector length 	<>

62 DSQ & VSQ	Square MPAC (precision D or V per Mode)		<>

63 SQRT & UNIT	Square root if D Mode, or UNIT if V Mode		<>

64 VDEF	Vector DEFine by popping Y and Z components		<>

65 NEGAT	NEGATe MPAC (precision D or T or V per Mode)		<>

66 TRIG	SIN, COS, ASIN, ACOS, etc. per parameter			<>

67 RTNM	ReTurn to Native Mode (but do not EXIT)			<A15>

70 PUSH	PUSH down data into stack (precision D or V per Mode) <>

71 PDDL	Push Down data as above and Double precision Load 	<A12>

72 PDVL	Push Down data as above and Vector Load			<A12>

73 POP	POP data off stack in next instruction (when needed) 	<>

74 GOTO	GOTO an entry point without affecting IRALIST		<A15>

75 CALL	CALL, pushing return address into IRALIST			<A15>

76 RTURN	ReTURN by popping return address off IRALIST		<>

77 EXIT	EXIT interpretive mode and continue in native mode 	<>

Short example of a calculation, x = (a/b)*(.25-c*d), showing how assembler pseudo-code is formatted according to interpreter protocols:

DLOAD	c		// Begin pseudo-code that expresses

DMP	d		// the calculation in the familiar

BDSU	.25DP		// assembly form, which the assembler

DMP	a		// rearranges into the triple-op

DDV	b		// formatting used by the interpreter,

STORE	x		// as shown below.

DLOAD	DMP	BDSU

C

D

.25 DP	// Addresses DATA=/DATAA pair in ROM

DMP	DDV

A

B

STORE	X

Note on Interpretive addressing: all instructions that read or store word data treat addresses below 0100 specially, using the address of the job’s work “VAC” area as a base register for those values. MPAC through PRIORITY are not part of VAC, but in a table. The topmost locations of the VAC are reserved for key variables as follows:

0077    S2	stepping amount for Index Register 2

0076	S1	stepping amount for Index Register 1

0075	X2	Index Register 2 (subtractive)

0074	X1	Index Register 1 (subtractive)

0072-3	ABV	Intermediate result (ABVAL) from UNIT vector operation

0070-1	VSQ	Intermediate result (ABVAL^2) from UNIT and ABVAL operations

0067	QPret	where EXIT will yield control to native-mode instructions

Within the remaining range, however, are two pushdown stacks, the data stack from addresses 0000 up to the maximum height the job requires, and the IRALIST return-address stack from 0066 down to whatever depth the job requires. So the middle range locations not thus reserved are available for general use.

Longer example of a calculation of the roots of a quadratic ax2 + bx + c, scaled thus:
r1 = (-b/2 + SQRT((b/2)2 - ac))/a and r2 = (-b/2 - SQRT((b/2)2 - ac))/a, showing how assembler “lines of code” presentation is reformatted according to interpreter protocols. In the address field, “-” signifies popping from the data stack.

QRoots	DLOAD	b

SR1			// b/2

NEGAT

PUSH			// -b/2 to stack location 00

DSQ

PUSH			// b^2/4 to stack location 02

DLOAD	a

DMP	c		// ac

BDSU	-		// b^2/4 – ac [pop 02]

SQRT

PUSH			// SQRT((b/2)^2 – ac) to stack 02

DAD	0		// -b/2 + SQRT((b/2)^2 – ac)

DDV	a		// (-b/2 + SQRT((b/2)^2 – ac))/a

STORE	r1

DLOAD	-		// SQRT((b/2)^2 – ac) [pop 02]

BDSU	-		// -b/2 - SQRT((b/2)^2 – ac) [pop 00]

DDV	a		// (-b/2 - SQRT((b/2)^2 – ac))/a

STORE r2

The assembler must reformat it thus:

QRoots	DLOAD	SHIFT	SR1	// b/2

b

NEGAT	PUSH	DSQ	// -b/2 to stack 00, (b/2)^2 = b^2/4

PDDL	DMP	BDSU	// b^2/4 to stack 02, ac,

a		// b^2/4 – ac [pop 02]

c

SQRT	PUSH	DAD	// SQRT(b^2/4 – ac) to stack 02,

0		// -b/2 + SQRT((b^2/4 – ac)

DDV	[NOP]	[NOP]	// (-b/2 + SQRT((b/2)^2 – ac))/a

a

STODL	r1		// SQRT((b/2)^2 – ac) [pop 02]

POP	BDSU	DDV	// -b/2 - SQRT((b/2)^2 – ac) [pop 00]

a		// (-b/2 - SQRT((b/2)^2 – ac))/a

STORE	r2

Annotations on the format, in which 18 lines of code are placed in just 14 words:

Shifts take two 6-bit groups, of which the second specifies the type and extent of shift.

The first BDSU pops because it sees an OPS word where its address would have been.

DAD 0 works because we know the data stack was empty at entry; if we hadn’t known what its state was, we’d have had to save the initial PushLoc in, say, InitPush, and write:
DAD *InitPush instead.

The “[NOP] [NOP]” pattern wouldn’t appear in a formatted listing. It appears here just to show how those 12 zero bits indicate to the interpreter that there are no more op codes in that OPS word after the DDV. Seems a pity to leave those 2 ops unused, but we don’t have a viable alternative here.

POP has to appear literally to make the second BDSU pop rather than using the DDV’s address, a.

Sample Pulse Sequences

In all but a few special cases, Time 6 (the final microsecond of each MCT) is implicit, either using staging code bits to switch to the next MCT of the current instruction, or copying the next instruction from the fixed-memory local register GF into the sequence control register SQ and the general buffer register B (control pulse F2SQB). Stage 0 is a universal prologue for indirect addressing. When there is no indirect addressing, the sequence begins with Stage 1.

INDEX1 – Index next instruction

1. RUXB WSE Z2SF         // Address from U if indexed, else from B.

2.

3. RE WY                 // Amount by which to index next instr.

4.

5. INCZ NISQ             // Standard proceed-to-next-instruction.

6. F2SQBX NDXT           // Special: add and flag, blocking *INCs.

DATAA1 – Place immediate data into A; set CC per A

- RUXB WA WCC Z2SF     // Address field to A and Condition.

- // Note: indexing DATAA may be useful.

- INCZ NISQ

INDIR0 - Indirect-address prologue. Setting BR1 & BR2

- RUXB WSE             // Yes, indexed indirect may be useful.

- RE WY DPCI           // For double precision instructions only,

- RU WB                // convert big-endian to little-endian.

- BR1 BR2 ST1          // 2 BRs block central register addressing.

PRET1 – Fetch one interpretive instruction from ROM

- RUXB WSF

- ST2

PRET2 – Set up AL for pret & fetch next native instruction

- F2ALCC Z2SF           // F19→A16, F18-13→A6-1, F12-1→L12-1.

- INCZ NISQ

EXEC1 – Perform one native instruction from ROM

- RUXB WSF

- NISQ                 // Secret of EXEC is leaving Z unchanged.

CALL1 – Call program module in ROM

- RUXB WSF

- RZ WQ                // Functions as a CALL if STQ at beginning

- RB WZ                // of module; else functions as a GOTO.

- INCZ NISQ

IFPNZ1 – Branch on CC Positive Non-Zero (REVIF => IFN|Z)

1. C=1xxx RUXB WSF       // Use effective address if branching,

1. C=0xxx Z2SF           // else ignore address and carry on.

2. C=1xxx RZ WQ          // All IF types set Q like CALL.

3. C=1xxx RB WZ

4.

5. INCZ NISQ

IFNZ1 – Branch on CC Non-Zero (REVIF => IFZER)

1. C=x1xx RUXB WSF

1. C=x0xx Z2SF

2. C=x1xx RZ WQ

3. C=x1xx RB WZ

4.

5. INCZ NISQ

IFNEG1 – Branch on CC Negative (REVIF => IFP|Z)

1. C=xx1x RUXB WSF

1. C=xx0x Z2SF

2. C=xx1x RZ WQ

3. C=xx1x RB WZ

4.

5. INCZ NISQ

IFNOV1 – Branch on CC No OVerflow (REVIF => IFOV)

1. C=xxx1 RUXB WSF

1. C=xxx0 Z2SF

2. C=xxx1 RZ WQ

3. C=xxx1 RB WZ

4.

5. INCZ NISQ

RUPT1 – Take highest priority interrupt [TO BE REWRITTEN]

1. IW  RUXB WSF

1. NIW Z2SF

2. IW  RZ WQ

3. IW  RB WZ

4.

5. INCZ NISQ

LDA1 – Load A and set CC

1. RUXB WSE Z2SF

2. B=00 RA WCC          // LDA A = TA: just test it.

2. B=01 RL WA WCC       // LDA L & LDA Q work as expected.

2. B=10 RQ WA WCC

3. B=11 RE WA WCC       // Indexed, indirect, or addr > 2.

4.

5. INCZ NISQ

LDL1 – Load L and set CC

1. RUXB WSE Z2SF

2. B=00 RA WL WCC       // LDL A & LDL Q work as expected.

2. B=01 RL WCC          // LDL L = TL: just test it.

2. B=10 RQ WL WCC

3. B=11 RE WL WCC       // Indexed, indirect, or addr > 2.

4.

5. INCZ NISQ

DLD1 – Double Load, L part or DT if direct address < 3

1. B=11 RUXB WSE WQ

1. B<11 Z2SF

2. B=11 RQ WY MONEX

2. B<11 RL WCC          // Any address 0-2 treated as DLD A.

3. B=11 RE WL WCC

3. B<11 RA WCCD         // DLD A = DT: Test DP value in AL.

4.                      // Assumes C(A,L) is sign-aligned.

5. B=11 RU WB ST2

5. B<11 INCZ NISQ

DLD2 – Double Load, A part and set CC for the combination

1. RB WSE Z2SF

2.

3. RE WY WCCD TSDAL     // May have to sign-align new data.

4. ADJA

5. RU WA INCZ NISQ

LDQ1 – Load Q and set CC

1. RUXB WSE Z2SF

2. B=00 RA WQ WCC       // LDQ A & LDQ L work as expected.

2. B=01 RL WQ WCC

2. B=10 RQ WCC          // LDQ Q = TQ: just test it.

3. B=11 RE WL WCC       // Indexed, indirect, or addr > 2.

4.

5. INCZ NISQ

R Lots of problems here with (dir) CR addressing and special cases

STA1 – Store A, or Zero A if direct address = 0

1. RUXB WSE Z2SF

2. B=00 WA WCC          // STA A = ZA: just zero it.

2. B=01 RA WL WCC       // STA L & STA Q work as expected.

2. B=10 RA WQ WCC

3.

4. B=11 RA WE WCC       // Indexed, indirect, or addr > 2.

5. INCZ NISQ

STL1 – Store L, or Zero L if direct address = 1

1. RUXB WSE Z2SF

2. B=00 RL WA WCC       // STL A & STL Q work as expected.

2. B=01 WL WCC          // STL L = ZL: just zero it.

2. B=10 RL WQ WCC

3.

4. B=11 RQ WE WCC       // Indexed, indirect, or addr > 2.

5. INCZ NISQ

DST1 – Double Store, L part; DZ case when dir addr < 3

1. B=11 RUXB WSE WQ     // Borrow Q as buffer to get upper.

1. B<11 Z2SF            // No ST2 cycle req’d when doing DZ.

2. B=11 RQ WY MONEX TSDAL // Signs of A, L set INVSL, ADJA

2. B<11 WA WL WCC       // Any address 0-2 treated as ZD.

3. B=11 INVSL

4. B=11 RL WE WCC       // Store sign-aligned L this cycle.

5. B=11 RU WB ST2

5. B<11 INCZ NISQ

DST2 – Double Store, A part

1. Z2SF RB WSE

2. RA WY ADJA           // ±1, or 0, to X according to TSDAL.

3.

4. RU WA WE WCCD        // Keep sign-aligned C(A,L).

5. INCZ NISQ

STQ1 – Store Q, or Zero Q if direct address = 2

1. RUXB WSE Z2SF

2. B=00 RQ WA WCC       // STQ A & STQ L work as expected.

2. B=01 RQ WL WCC

2. B=10 WQ WCC          // STQ Q = ZQ: just zero it.

3.

4. B=11 RQ WE WCC       // Indexed, indirect, or addr > 2.

5. INCZ NISQ

XCA1 – Exchange A

1. Z2SF RB WSE

2. 0xx RA WY TCR

2. 1xx RA WY

3. RGE WA WCC

4. RU WGE

5. INCZ NISQ

XCL1 – Exchange L

1. Z2SF RB WSE

2. RL WB

3. RGE WL WCC

4. RB WGE

5. INCZ NISQ

DXC1 – Double Exchange, L part

1. RB WSE WY MONEX TSDAL

2. 0x RL WB

2. 1x RL WB INVSL

3. RGE WL WCC

4. RB WGE

5. RU WB ST2

DXC2 – Double Exchange, A part

1. Z2SF RB WSE

2. 0x RA WY

2. 10 MONEX RA WY

2. 11 PONEX RA WY

3. RGE WA WCCD

4. RU WGE

5. INCZ NISQ

XCQ1 – Exchange Q

- Z2SF RB WSE

- RQ WB

- RGE WQ WCC

- RB WGE

- INCZ NISQ

Logic for NORM: Normalize Signed Double Precision Number in A and L

NORM shifts C(AL) numerically until it is normalized with no overflow present (A16=A15≠A14), even if overflow is initially present, and places in Q the shift count required to do this. If C(AL) is zero, no shifting occurs, the zero condition is set, Q = 0.

The duration is 6 PT = 1 MCT if C(AL) is initially normalized or nearly so. Otherwise each shift beyond the first two takes 2 additional PT. Like all shift instructions, this one can insert a sub-MCT gap into the succession of MCTs.

Fetching next instruction begins (Z2SF) unconditionally in PT1. Also in PT1, Q = -1 and C(AL) shifted right 1 copies to ALS, CC reflects AL.

In PT2, if CC/OV then C(ALS) copies to AL and NISQ, else adder inputs X & Y are zeroed. This occurrence of Next Instruction to SQ (NISQ) has a delayed effect (to PT6) because a ROM cycle is in progress; in the intervening pulse times no actions occur.

In PT3, C(AL) shifted left 1 copies to ALS, adder sum U goes to Q (value zero in first occurrence of PT3), CC reflects new ALS.

In PT4, if CC/OV then CC reflects AL, NISQ (delayed if first PT4), else C(ALS) copies unshifted to AL, Q enters adder to be incremented, force PT3 as next step.

No pulses are specified for PT5, but when PT4 takes the else branch, PT3 repeats: C(AL) shifted left 1 copies to ALS, adder sum U goes to Q (an increase of 1 in non-first PT3), CC reflects new ALS.

No pulses are specified for PT6, but when PT3 logic was repeated in occurred in PT5, PT4 repeats: if CC/OV then CC reflects AL, NISQ (here not delayed, because the ROM cycle is complete), else C(ALS) copies unshifted to AL, Q enters the adder to be incremented, force PT3 as the next step.

The logic of PT3 and PT4 repeats until normalization is achieved, and the first NISQ is effective at once. Thus, the duration of NORM, and the shift count in Q, can be:

6 PT when Q = -1, 0, or 1 (1 MCT)

8 PT when Q = 2

10 PT when Q = 3

12 PT when Q = 4 (2 MCT)

. . . . . . . . . . . . . . .

60 PT when Q = 28 (10 MCT)

61 PT when Q = 29

That maximum may be be the longest instruction duration in Block III, but it’s not seriously out of line. Statistically, 1 or <2 MCT will be predominant, especially when normalizing data obtained from the Interrupt Request (IR) register while entering an interrupt. NORM plays a key role in identifying the highest-priority interrupt request.

Instruction Sequencing Logic, Including IFs, RUPT, and Cycle Steals

Most instructions, most of the time, will be completed in one Memory Cycle Time, specifically those whose data-handling functions can be completed in one cycle of RAM/erasable. In these cases, Pulse Time 1 copies the data address to SE and the next-instruction address (usually from Z) to SF.

In all cases, pulse NISQ (Next Instruction to SQ) establishes the current memory cycle as the last one in the current instruction. Usually, it is coded in Pulse Time 6, where it advances the operation code to the diode-matrix SeQuence generator where it will become effective in the following PT1. However, other pulses coded into PT6 of the current instruction will function. In some cases, like NORM when 2 or more shifts are needed, or other shift instructions when the number of shifts exceeds the capacity of one MCT, NISQ may appear in earlier Pulse Times. Those NISQs are effective immediately when no ROM or RAM cycle is in progress, but if a memory cycle is in progress, the effect is held until PT6 of that cycle; moreover, any pulses coded into Pulse Times after the NISQ but in or preceding PT6 are blocked and ineffective.

When a NISQ pulse becomes effective, what it puts into SQ can be overridden by the highest-priority cycle-steal request, so that the “stolen” RAM cycle occurs between instructions and can use data-handling parts of the ALU. For that reason, there is an exception: if any inputs to X or Y have been done after the most recent RU pulse, such cycle steals are blocked because the adder output U will be needed in the next instruction.

Whenever a ROM/fixed cycle begins, i.e. when an address is written into SF, dedicated incrementer logic begins, so that pulse RSF (Read SF) will collect the next higher address for other uses. Tentatively, RSF will be available from PT3 onward in that cycle, to allow a simple incrementer without explicit-carry logic. Hopefully, PT2 will turn out to work equally well. Usually, the address read by RSF will be written into Z, setting it up for use in the next cycle.

Any IF-type instruction, when its PT1 finds the stated condition unsatisfied, operates as a NOP, copying Z to SF and the incremented version back to Z like any ordinary instruction.

Actual transfers of control (except for EXEC and RUPT) work a little differently. PT1 copies the instruction’s address field into SF, also copying Z into Q as a possible return address before RSF updates Z. Because any transfer of control may turn out to be a CALL rather than a GOTO, the Return Address Pointer, RAP, is placed into Y and a carry-in ordered to increment it, but the sum is not read out (by RU) in this cycle. Instead, the sum is carried over into the first cycle of the instruction to which control is being transferred, blocking intervening cycle steals as mentioned above. We’ll get back to what happens there in a minute.

EXEC also copies its address field into SF in PT1 to start the fetching of the instruction being EXECuted, but places that address’s incremented form into LS instead of Z. By leaving Z unchanged, it sets up the instruction after the EXEC to be the successor to the EXECuted instruction (unless it performs a successful transfer of control). Then it performs the same “unfinished business” logic on RAP as in any other transfer of control, blocking cycle steal(s) between the EXEC and the instruction being EXECuted. Note that EXEC passes more data to its target instruction than other transfers: Q as well as A and L.

RUPT is a special transfer of control that needs no address field because it can transfer control only to ROM location 00001. If its PT1 finds IR to be all zero, the copying of Z to SF and putting the incremented value into Z occurs as in ordinary instructions, and no transfer of control takes place. But when PT1 sees any non-zero bits in IR, it zeros both SE and SF, copies Z into adder input Y, and starts a decrement. The 0000 in SE addresses QRUPT, into which Q is saved, after which the sum, b(Z)-1, goes into Q to establish the address of the RUPT itself as the resume address. Then it performs the same “unfinished business” logic on RAP as in any other transfer of control, blocking cycle steal(s) between the RUPT and the instruction (mandated to be a PRA) in 00001.

Almost all of this RUPT logic is shared with the Copy Block (CPBK) instruction, using the same test of IR at the beginning of each of its read cycles to switch its action to the RUPT sequence given here. When IR is found to be zero, CPBK’s two RAM cycles (read from source and write into destination) continue normally, incrementing the address pointers in A and L, and decrementing and testing Q to see whether the block transfer is finished. This allows interrupts to be taken at any point in the block transfer, such that resuming after each interrupt executes the CPBK again, but with its counters set to pick up where it left off. That works because Q is saved in QRUPT as described above, and because A and L are saved at the interrupt entry point (by mandate) and restored just before the resume. All 3 of those built-in register saves are peculiar to RUPT, and the A and L restores are not done in regular call-return sequences. The reason the RUPT logic is parallelled with the read cycle and not the write cycle is that a CPBK preceded by a DATA= code omits read cycles and just puts the immediate data into all the destination locations without allowing any interrupts. Letting it allow such interrupts would require more logic to retain the immediate-data state through each interrupt.

The question of whether a transfer of control is a CALL or a GOTO is decided at the entry-point instruction receiving control. If a CALL is intended, the entry-point instruction itself must be Push Return Address (PRA), a no-address type. In PT1, PRA copies the adder sum U into RAP (completing the extension of the RA list by one word) and into SE (beginning the cycle that fills the new head of the RA list)—also finishing the “unfinished business” from the transfer-control instruction. Then it copies Q to the RAM location thus addressed, populating the new head. It also performs SAVCC.

The ReturnAddress (RA) list occupies RAM locations 0001 up to (at most) 0077. Thus a 0000 in RAP indicates an empty list, ready for the increment that occurs when an RA is pushed onto the list. Location 0000 itself is dedicated to interrupt storage QRUPT.

What follows the PRA is whatever processing the subroutine has to do, including any CALLs to lower-level subroutines it requires. Returning from the subroutine can be done from any number of places within it, by any successful transfer of control instruction (conditional or unconditional) with an address of 00000. The all-zero address field is recognized as a special case to invoke the Return logic, which starts by copying RAP to SE and to Y for decrementing. Then it puts the adder result into RAP, popping the head off the Return-Address list. The address read from RAM goes into Z, and the next cycle

returns control to the calling routine. It copies Z into SF to start the read, copies the incremented version back into Z in the usual way, and copies the permanent QRUPT address (0000) into SE and thus restores Q as required for resuming after an interrupt, though that is not mandated for other returns. However, it is beneficial if the original call was preceded by STQ QRUPT, which is legitimate in any non-task code. Note also that returning from code invoked by RUPT or CPBK (i.e. the RSM instruction as shown below) gives control back to the same RUPT or CPBK so that it can again check IR for Interrupt Requests and invoke the highest-priority of those before continuing the current Job. RSM differs from RTURN (assembled as GOTO 00000) only in adding restoration of the saved Condition Code.

The entry point targeted by EXEC will usually be a one-instruction subroutine, often enough a DATA= or DATAA, simply using the undisturbed state of Z to return control to the instruction following the EXEC. Usually, the EXEC will be INDEXed. Sometimes one or a few of the possible targets for a given EXEC will be a routine that is not complete in one instruction, in which case it must be a CALL, as seen below.

Interrupt Processing

P01 Code, located starting at 00001, common to all interrupts. Takes

R02 15 MCT plus a little for possibly longer shifts, plus RuptX time.

03 RUPT     PRA              Convert to CALL; SAVCC

04          DST    ALRupt    Save A,L in case CPBK

05          READ   IR        Still has seen request

06          NORM             Find highest prio req

07          DATAA  Rupt0Com  57777 as is for RUPT 0

08          NDX    Q

09          SAR    0         Note sign extension

10          WAND   IR        Turn off RUPT request

11          NDX    Q

12          EXEC   RuptVect  NDX picks out a CALL

13          DLD    ALRupt    Restore in case CPBK

14          RSM              RTURN plus RSTCC

15 RuptVect CALL  Rupt0

16          CALL  Rupt1

17          CALL  Rupt2

. . . . . . . . . . . . . . . . . . . . . . .

nn          CALL  Ruptn

Change-of-Job Checking

T      NewJob    NZ when a higher pri-

IFPNZ  ChangJob   ority job is waiting